J1

sysname J1

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 10

interface Ethernet0/0/2

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/3

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCEC3794

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

J2

sysname J2

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 10

interface Ethernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/3

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCE30444

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

JX

sysname JX

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/3

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/4

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCAC4BE6

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

科研楼LSW

sysname KY

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 20

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCA12D91

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

1号学院楼

sysname X1

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 30

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC036E4A

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

行政楼

sysname XZ

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 40

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCC74F7A

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

图书馆

sysname TS

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 50

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCFD1786

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

实验楼

sysname SY

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 60

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCE30657

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

2号学院楼

sysname X2

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 70

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC35019D

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

AR7

[V200R003C00]

sysname XXZX-PC

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load portalpage.zip

drop illegal-mac alarm

set cpu-usage threshold 80 restore 75

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 192.168.120.1 255.255.255.0

ip route-static 0.0.0.0 0.0.0.0 192.168.120.254

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

LSW18

sysname LSW18

vlan batch 110 120

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 110

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC8879DE

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW19

sysname LSW19

vlan batch 110 120

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 120

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC183B8C

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW32

sysname LSW32

undo info-center enable

vlan batch 105 to 108 110 120

stp instance 3 root primary

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif105

ip address 192.168.1.18 255.255.255.252

interface Vlanif106

ip address 192.168.1.22 255.255.255.252

interface Vlanif107

ip address 192.168.1.26 255.255.255.252

interface Vlanif108

ip address 192.168.1.30 255.255.255.252

interface Vlanif110

ip address 192.168.110.252 255.255.255.0

vrrp vrid 110 virtual-ip 192.168.110.254

vrrp vrid 110 priority 120

interface Vlanif120

ip address 192.168.120.252 255.255.255.0

vrrp vrid 120 virtual-ip 192.168.120.254

vrrp vrid 120 priority 120

interface Eth-Trunk2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 106

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 105

stp disable

interface GigabitEthernet0/0/3

eth-trunk 2

interface GigabitEthernet0/0/4

eth-trunk 2

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

area 0.0.0.2

network 192.168.110.0 0.0.0.255

network 192.168.1.16 0.0.0.3

network 192.168.1.20 0.0.0.3

network 192.168.120.0 0.0.0.255

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC900A19

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW34

sysname LSW34

undo info-center enable

vlan batch 105 to 108 110 120

stp instance 3 root secondary

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif105

ip address 192.168.1.18 255.255.255.252

interface Vlanif106

ip address 192.168.1.22 255.255.255.252

interface Vlanif107

ip address 192.168.1.26 255.255.255.252

interface Vlanif108

ip address 192.168.1.30 255.255.255.252

interface Vlanif110

ip address 192.168.110.253 255.255.255.0

vrrp vrid 110 virtual-ip 192.168.110.254

vrrp vrid 110 priority 110

interface Vlanif120

ip address 192.168.120.253 255.255.255.0

vrrp vrid 120 virtual-ip 192.168.120.254

vrrp vrid 120 priority 110

interface Eth-Trunk2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 107

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 108

stp disable

interface GigabitEthernet0/0/3

eth-trunk 2

interface GigabitEthernet0/0/4

eth-trunk 2

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

default-route-advertise

area 0.0.0.2

network 192.168.120.0 0.0.0.255

network 192.168.1.24 0.0.0.3

network 192.168.1.28 0.0.0.3

network 192.168.110.0 0.0.0.255

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCBC3689

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW1

sysname LSW1

undo info-center enable

vlan batch 10 20 30 40 50 60 70 100 to 108

stp instance 1 root primary

stp instance 2 root secondary

dhcp enable

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

ip pool vlan10

gateway-list 10.10.10.254

network 10.10.10.0 mask 255.255.255.0

excluded-ip-address 10.10.10.252 10.10.10.253

dns-list 114.114.114.114

ip pool vlan20

gateway-list 10.10.20.254

network 10.10.20.0 mask 255.255.255.0

excluded-ip-address 10.10.20.252 10.10.20.253

dns-list 114.114.114.114

ip pool vlan30

gateway-list 10.10.30.254

network 10.10.30.0 mask 255.255.255.0

excluded-ip-address 10.10.30.252 10.10.30.253

dns-list 114.114.114.114

ip pool vlan40

gateway-list 10.10.40.254

network 10.10.40.0 mask 255.255.255.0

excluded-ip-address 10.10.40.252 10.10.40.253

dns-list 114.114.114.114

ip pool vlan50

gateway-list 10.10.50.254

network 10.10.50.0 mask 255.255.255.0

excluded-ip-address 10.10.50.252 10.10.50.253

dns-list 114.114.114.114

ip pool vlan60

gateway-list 10.10.60.254

network 10.10.60.0 mask 255.255.255.0

excluded-ip-address 10.10.60.252 10.10.60.253

dns-list 114.114.114.114

ip pool vlan70

gateway-list 10.10.70.254

network 10.10.70.0 mask 255.255.255.0

excluded-ip-address 10.10.70.252 10.10.70.253

dns-list 114.114.114.114

ip pool vlan100

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

excluded-ip-address 192.168.100.252 192.168.100.253

dns-list 114.114.114.114

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif10

ip address 10.10.10.252 255.255.255.0

vrrp vrid 10 virtual-ip 10.10.10.254

vrrp vrid 10 priority 120

dhcp select global

interface Vlanif20

ip address 10.10.20.252 255.255.255.0

vrrp vrid 20 virtual-ip 10.10.20.254

vrrp vrid 20 priority 120

dhcp select global

interface Vlanif30

ip address 10.10.30.252 255.255.255.0

vrrp vrid 30 virtual-ip 10.10.30.254

vrrp vrid 30 priority 120

dhcp select global

interface Vlanif40

ip address 10.10.40.252 255.255.255.0

vrrp vrid 40 virtual-ip 10.10.40.254

vrrp vrid 40 priority 110

dhcp select global

interface Vlanif50

ip address 10.10.50.252 255.255.255.0

vrrp vrid 50 virtual-ip 10.10.50.254

vrrp vrid 50 priority 110

dhcp select global

interface Vlanif60

ip address 10.10.60.252 255.255.255.0

vrrp vrid 60 virtual-ip 10.10.60.254

vrrp vrid 60 priority 110

dhcp select global

interface Vlanif70

ip address 10.10.70.252 255.255.255.0

vrrp vrid 70 virtual-ip 10.10.70.254

vrrp vrid 70 priority 110

dhcp select global

interface Vlanif100

ip address 192.168.100.252 255.255.255.0

vrrp vrid 100 virtual-ip 192.168.100.254

vrrp vrid 100 priority 120

dhcp select global

interface Vlanif101

ip address 192.168.1.1 255.255.255.252

interface Vlanif102

ip address 192.168.1.5 255.255.255.252

interface Vlanif106

ip address 192.168.1.21 255.255.255.252

interface Vlanif107

ip address 192.168.1.25 255.255.255.252

interface Eth-Trunk1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 101

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 102

stp disable

interface GigabitEthernet0/0/3

eth-trunk 1

interface GigabitEthernet0/0/4

eth-trunk 1

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/7

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/8

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/9

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/12

port link-type access

port default vlan 106

stp disable

interface GigabitEthernet0/0/13

port link-type access

port default vlan 107

stp disable

ospf 1

area 0.0.0.0

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

network 0.0.0.0 255.255.255.255

area 0.0.0.1

network 10.10.10.0 0.0.0.255

network 10.10.20.0 0.0.0.255

network 10.10.30.0 0.0.0.255

network 10.10.40.0 0.0.0.255

network 10.10.50.0 0.0.0.255

network 10.10.60.0 0.0.0.255

network 10.10.70.0 0.0.0.255

network 192.168.100.0 0.0.0.255

area 0.0.0.2

network 192.168.1.20 0.0.0.3

network 192.168.1.24 0.0.0.3

ospf 2

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC82448F

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW2

sysname LSW2

undo info-center enable

vlan batch 10 20 30 40 50 60 70 100 to 108

stp instance 1 root secondary

stp instance 2 root primary

dhcp enable

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

ip pool vlan10

gateway-list 10.10.10.254

network 10.10.10.0 mask 255.255.255.0

excluded-ip-address 10.10.10.252 10.10.10.253

dns-list 114.114.114.114

ip pool vlan20

gateway-list 10.10.20.254

network 10.10.20.0 mask 255.255.255.0

excluded-ip-address 10.10.20.252 10.10.20.253

dns-list 114.114.114.114

ip pool vlan30

gateway-list 10.10.30.254

network 10.10.30.0 mask 255.255.255.0

excluded-ip-address 10.10.30.252 10.10.30.253

dns-list 114.114.114.114

ip pool vlan40

gateway-list 10.10.40.254

network 10.10.40.0 mask 255.255.255.0

excluded-ip-address 10.10.40.252 10.10.40.253

dns-list 114.114.114.114

ip pool vlan50

gateway-list 10.10.50.254

network 10.10.50.0 mask 255.255.255.0

excluded-ip-address 10.10.50.252 10.10.50.253

dns-list 114.114.114.114

ip pool vlan60

gateway-list 10.10.60.254

network 10.10.60.0 mask 255.255.255.0

excluded-ip-address 10.10.60.252 10.10.60.253

dns-list 114.114.114.114

ip pool vlan70

gateway-list 10.10.70.254

network 10.10.70.0 mask 255.255.255.0

excluded-ip-address 10.10.70.252 10.10.70.253

dns-list 114.114.114.114

ip pool vlan100

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

excluded-ip-address 192.168.100.252 192.168.100.253

dns-list 114.114.114.114

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif10

ip address 10.10.10.253 255.255.255.0

vrrp vrid 10 virtual-ip 10.10.10.254

vrrp vrid 10 priority 110

dhcp select global

interface Vlanif20

ip address 10.10.20.253 255.255.255.0

vrrp vrid 20 virtual-ip 10.10.20.254

vrrp vrid 20 priority 110

dhcp select global

interface Vlanif30

ip address 10.10.30.253 255.255.255.0

vrrp vrid 30 virtual-ip 10.10.30.254

vrrp vrid 30 priority 110

dhcp select global

interface Vlanif40

ip address 10.10.40.253 255.255.255.0

vrrp vrid 40 virtual-ip 10.10.40.254

vrrp vrid 40 priority 120

dhcp select global

interface Vlanif50

ip address 10.10.50.253 255.255.255.0

vrrp vrid 50 virtual-ip 10.10.50.254

vrrp vrid 50 priority 120

dhcp select global

interface Vlanif60

ip address 10.10.60.253 255.255.255.0

vrrp vrid 60 virtual-ip 10.10.60.254

vrrp vrid 60 priority 120

dhcp select global

interface Vlanif70

ip address 10.10.70.253 255.255.255.0

vrrp vrid 70 virtual-ip 10.10.70.254

vrrp vrid 70 priority 120

dhcp select global

interface Vlanif100

ip address 192.168.100.253 255.255.255.0

vrrp vrid 100 virtual-ip 192.168.100.254

vrrp vrid 100 priority 110

dhcp select global

interface Vlanif103

ip address 192.168.1.9 255.255.255.252

interface Vlanif104

ip address 192.168.1.13 255.255.255.252

interface Vlanif105

ip address 192.168.1.17 255.255.255.252

interface Vlanif108

ip address 192.168.1.29 255.255.255.252

interface Eth-Trunk1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 104

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 103

stp disable

interface GigabitEthernet0/0/3

eth-trunk 1

interface GigabitEthernet0/0/4

eth-trunk 1

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/7

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/8

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/9

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/12

port link-type access

port default vlan 105

stp disable

interface GigabitEthernet0/0/13

port link-type access

port default vlan 108

stp disable

interface GigabitEthernet0/0/14

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

area 0.0.0.0

network 192.168.1.8 0.0.0.3

network 192.168.1.12 0.0.0.3

area 0.0.0.1

network 10.10.10.0 0.0.0.255

network 10.10.20.0 0.0.0.255

network 10.10.30.0 0.0.0.255

network 10.10.40.0 0.0.0.255

network 10.10.50.0 0.0.0.255

network 10.10.60.0 0.0.0.255

network 10.10.70.0 0.0.0.255

network 192.168.100.0 0.0.0.255

area 0.0.0.2

network 192.168.1.16 0.0.0.3

network 192.168.1.28 0.0.0.3

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC416636

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

FW1

!Software Version V500R005C10SPC300

!Last configuration was saved at 2026-06-06 07:08:19 UTC

sysname FW1

l2tp domain suffix-separator @

undo info-center enable

ipsec sha2 compatible enable

undo telnet ipv6 server enable

hrp enable

hrp interface GigabitEthernet1/0/0 remote 1.1.1.2

hrp mirror session enable

hrp load balance device

update schedule location-sdb weekly Sun 07:23

firewall defend action discard

banner enable

user-manage web-authentication security port 8887

undo privacy-statement english

undo privacy-statement chinese

page-setting

user-manage security version tlsv1.1 tlsv1.2

password-policy

level high

user-manage single-sign-on ad

user-manage single-sign-on tsm

user-manage single-sign-on radius

user-manage auto-sync online-user

web-manager security version tlsv1.1 tlsv1.2

web-manager enable

web-manager security enable

firewall dataplane to manageplane application-apperceive default-action drop

undo ips log merge enable

decoding uri-cache disable

update schedule ips-sdb daily 02:04

update schedule av-sdb daily 02:04

update schedule sa-sdb daily 02:04

update schedule cnc daily 02:04

update schedule file-reputation daily 02:04

ip vpn-instance default

ipv4-family

time-range worktime

period-range 08:00:00 to 18:00:00 working-day

ike proposal default

encryption-algorithm aes-256 aes-192 aes-128

dh group14

authentication-algorithm sha2-512 sha2-384 sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

aaa

authentication-scheme default

authentication-scheme admin_local

authentication-scheme admin_radius_local

authentication-scheme admin_hwtacacs_local

authentication-scheme admin_ad_local

authentication-scheme admin_ldap_local

authentication-scheme admin_radius

authentication-scheme admin_hwtacacs

authentication-scheme admin_ad

authorization-scheme default

accounting-scheme default

domain default

service-type internetaccess ssl-vpn l2tp ike

internet-access mode password

reference user current-domain

manager-user audit-admin

password cipher @%@%|6S9U>C3D%*9!eN,94t/2D-XuqMS8igj9Xu@^s0fq#&LD-[2@%@%

service-type web terminal

level 15

manager-user api-admin

password cipher @%@%Id]R<~fBU=6(&s$Ccb#NTgM5[qv+&%>W<M+TQ+7/r|9XgM8T@%@%

level 15

manager-user admin

password cipher @%@%KC"',0gD~56ecF@@L'!ItnY{kFUJU6Hm`~(x$[J\TB!1nY!t@%@%

service-type web terminal telnet

level 15

role system-admin

role device-admin

role device-admin(monitor)

role audit-admin

bind manager-user audit-admin role audit-admin

bind manager-user admin role system-admin

l2tp-group default-lns

interface GigabitEthernet0/0/0

undo shutdown

ip binding vpn-instance default

ip address 192.168.0.1 255.255.255.0

alias GE0/METH

interface GigabitEthernet1/0/0

undo shutdown

ip address 1.1.1.1 255.255.255.252

interface GigabitEthernet1/0/1

undo shutdown

ip address 192.168.1.2 255.255.255.252

interface GigabitEthernet1/0/2

undo shutdown

ip address 192.168.1.10 255.255.255.252

interface GigabitEthernet1/0/3

undo shutdown

ip address 192.168.200.252 255.255.255.0

vrrp vrid 2 virtual-ip 192.168.200.254 active

interface GigabitEthernet1/0/4

undo shutdown

ip address 100.1.1.2 255.255.255.248

vrrp vrid 1 virtual-ip 100.1.1.1 active

service-manage ping permit

interface GigabitEthernet1/0/5

undo shutdown

ip address 2.2.2.2 255.255.255.248

vrrp vrid 2 virtual-ip 2.2.2.1 active

interface GigabitEthernet1/0/6

undo shutdown

interface Virtual-if0

interface NULL0

interface Tunnel0

interface Tunnel1

ip address 192.168.1.17 255.255.255.252

tunnel-protocol gre

source 100.1.1.2

destination 116.116.116.2

service-manage ping permit

interface Tunnel2

ip address 192.168.1.21 255.255.255.252

tunnel-protocol gre

source 2.2.2.2

destination 117.117.117.2

service-manage ping permit

firewall zone local

set priority 100

firewall zone trust

set priority 85

add interface GigabitEthernet0/0/0

add interface GigabitEthernet1/0/1

add interface GigabitEthernet1/0/2

firewall zone untrust

set priority 5

add interface GigabitEthernet1/0/4

add interface GigabitEthernet1/0/5

add interface Tunnel1

add interface Tunnel2

firewall zone dmz

set priority 50

add interface GigabitEthernet1/0/0

add interface GigabitEthernet1/0/3

ospf 1

default-route-advertise always

silent-interface GigabitEthernet1/0/3

area 0.0.0.0

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

network 192.168.1.8 0.0.0.3

network 192.168.1.12 0.0.0.3

network 192.168.1.20 0.0.0.3

network 192.168.200.0 0.0.0.255

ip route-static 0.0.0.0 0.0.0.0 100.1.1.6

ip route-static 0.0.0.0 0.0.0.0 2.2.2.6 preference 80

ip route-static 10.20.0.0 255.255.0.0 192.168.1.18

snmp-agent

snmp-agent local-engineid 800007DB0300E0FC465C76

snmp-agent community read cipher %^%#J~o#R2]=m::30[Kf-re3waFY>3,#o=DGv}TnErkRZ=a}-kTZjV5(gG,077w4,9rPU_OTTR$*xoR%nQ,S%^%#

snmp-agent sys-info version v2c v3

snmp-agent trap enable

undo ssh server compatible-ssh1x enable

ssh authentication-type default password

ssh server cipher aes256_ctr aes128_ctr

ssh server hmac sha2_256 sha1

ssh client cipher aes256_ctr aes128_ctr

ssh client hmac sha2_256 sha1

firewall detect ftp

nat server server6_all protocol tcp global 202.96.37.121 www inside 192.168.200.2 www

user-interface con 0

authentication-mode aaa

user-interface vty 0 4

authentication-mode aaa

user-interface vty 16 20

pki realm default

sa

location

nat address-group l 0

mode pat

nat address-group 1 1

mode pat

section 0 202.96.37.88 202.96.37.88

nat address-group 0 2

mode pat

section 0 202.96.37.20 202.96.37.30

multi-linkif

mode proportion-of-weight

right-manager server-group

device-classification

device-group pc

device-group mobile-terminal

device-group undefined-group

user-manage server-sync tsm

security-policy

rule name trust-dmz

source-zone trust

destination-zone dmz

source-address 10.10.40.0 mask 255.255.255.0

destination-address 192.168.200.1 mask 255.255.255.255

action permit

rule name trust-untrust

source-zone trust

destination-zone untrust

source-address-exclude 10.10.70.0 mask 255.255.255.0

action permit

rule name untrust-dmz

source-zone untrust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

rule name local2any

source-zone local

action permit

rule name any2local

destination-zone local

action permit

auth-policy

traffic-policy

policy-based-route

nat-policy

rule name nat

source-zone trust

destination-zone untrust

source-address 10.10.0.0 mask 255.255.192.0

source-address 192.168.110.0 mask 255.255.255.0

source-address 192.168.120.0 mask 255.255.255.0

destination-address-exclude 10.20.0.0 mask 255.255.0.0

destination-address-exclude 10.30.0.0 mask 255.255.0.0

action source-nat address-group 0

quota-policy

pcp-policy

dns-transparent-policy

rightm-policy

return

FW2

!Software Version V500R005C10SPC300

!Last configuration was saved at 2026-06-06 07:08:30 UTC

sysname FW2

l2tp domain suffix-separator @

undo info-center enable

ipsec sha2 compatible enable

undo telnet server enable

undo telnet ipv6 server enable

hrp enable

hrp interface GigabitEthernet1/0/0 remote 1.1.1.1

hrp mirror session enable

hrp load balance device

update schedule location-sdb weekly Sun 01:54

firewall defend action discard

banner enable

user-manage web-authentication security port 8887

undo privacy-statement english

undo privacy-statement chinese

page-setting

user-manage security version tlsv1.1 tlsv1.2

password-policy

level high

user-manage single-sign-on ad

user-manage single-sign-on tsm

user-manage single-sign-on radius

user-manage auto-sync online-user

web-manager security version tlsv1.1 tlsv1.2

web-manager enable

web-manager security enable

firewall dataplane to manageplane application-apperceive default-action drop

undo ips log merge enable

decoding uri-cache disable

update schedule ips-sdb daily 04:46

update schedule av-sdb daily 04:46

update schedule sa-sdb daily 04:46

update schedule cnc daily 04:46

update schedule file-reputation daily 04:46

ip vpn-instance default

ipv4-family

time-range worktime

period-range 08:00:00 to 18:00:00 working-day

ike proposal default

encryption-algorithm aes-256 aes-192 aes-128

dh group14

authentication-algorithm sha2-512 sha2-384 sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

aaa

authentication-scheme default

authentication-scheme admin_local

authentication-scheme admin_radius_local

authentication-scheme admin_hwtacacs_local

authentication-scheme admin_ad_local

authentication-scheme admin_ldap_local

authentication-scheme admin_radius

authentication-scheme admin_hwtacacs

authentication-scheme admin_ad

authorization-scheme default

accounting-scheme default

domain default

service-type internetaccess ssl-vpn l2tp ike

internet-access mode password

reference user current-domain

manager-user audit-admin

password cipher @%@%CrgBRB69+>ELs0RulZ[V.Q}Ljvoz14tFP1^}pQ%u}``UQ}O.@%@%

service-type web terminal

level 15

manager-user api-admin

password cipher @%@%bna.T5B!sIxA4t,)D^yQ<:e;6L'~Ef:=1HXOW\<>EUO,:e><@%@%

level 15

manager-user admin

password cipher @%@%euP1JP:N;;.:vh82al:$x~X`4R[yFeJP_YzVMU<91lk#~Xcx@%@%

service-type web terminal telnet

level 15

role system-admin

role device-admin

role device-admin(monitor)

role audit-admin

bind manager-user audit-admin role audit-admin

bind manager-user admin role system-admin

l2tp-group default-lns

interface GigabitEthernet0/0/0

undo shutdown

ip binding vpn-instance default

ip address 192.168.0.1 255.255.255.0

alias GE0/METH

interface GigabitEthernet1/0/0

undo shutdown

ip address 1.1.1.2 255.255.255.252

interface GigabitEthernet1/0/1

undo shutdown

ip address 192.168.1.6 255.255.255.252

interface GigabitEthernet1/0/2

undo shutdown

ip address 192.168.1.14 255.255.255.252

interface GigabitEthernet1/0/3

undo shutdown

ip address 192.168.200.253 255.255.255.0

vrrp vrid 2 virtual-ip 192.168.200.254 standby

service-manage ping permit

interface GigabitEthernet1/0/4

undo shutdown

ip address 100.1.1.3 255.255.255.248

vrrp vrid 1 virtual-ip 100.1.1.1 standby

service-manage ping permit

interface GigabitEthernet1/0/5

undo shutdown

ip address 2.2.2.3 255.255.255.248

vrrp vrid 1 virtual-ip 2.2.2.1 standby

interface GigabitEthernet1/0/6

undo shutdown

interface Virtual-if0

interface NULL0

interface Tunnel0

interface Tunnel1

ip address 192.168.1.17 255.255.255.252

tunnel-protocol gre

source 202.96.37.10

destination 116.116.116.2

service-manage ping permit

interface Tunnel2

ip address 192.168.1.21 255.255.255.252

tunnel-protocol gre

source 202.96.37.10

destination 117.117.117.2

service-manage ping permit

firewall zone local

set priority 100

firewall zone trust

set priority 85

add interface GigabitEthernet0/0/0

add interface GigabitEthernet1/0/1

add interface GigabitEthernet1/0/2

firewall zone untrust

set priority 5

add interface GigabitEthernet1/0/4

add interface GigabitEthernet1/0/5

add interface Tunnel1

add interface Tunnel2

firewall zone dmz

set priority 50

add interface GigabitEthernet1/0/0

add interface GigabitEthernet1/0/3

ospf 1

default-route-advertise always

import-route static

silent-interface GigabitEthernet1/0/3

area 0.0.0.0

network 192.168.1.4 0.0.0.3

network 192.168.1.12 0.0.0.3

network 192.168.200.0 0.0.0.255

ip route-static 0.0.0.0 0.0.0.0 202.96.37.3

ip route-static 0.0.0.0 0.0.0.0 100.1.1.6

ip route-static 0.0.0.0 0.0.0.0 2.2.2.6 preference 80

ip route-static 10.20.0.0 255.255.0.0 192.168.1.18

ip route-static 10.30.0.0 255.255.0.0 192.168.1.22

snmp-agent

snmp-agent local-engineid 800007DB0300E0FC657F13

snmp-agent community read cipher %^%#r*dK@5*1AG%nnV9AmI':S$p(3X^I#Q,+"v'XbuiU2C~5V/3xL)l+';+N{u6V+1a\VJ'-W&(KR6<.0y_M%^%#

snmp-agent sys-info version v2c v3

snmp-agent trap enable

undo ssh server compatible-ssh1x enable

ssh authentication-type default password

ssh server cipher aes256_ctr aes128_ctr

ssh server hmac sha2_256 sha1

ssh client cipher aes256_ctr aes128_ctr

ssh client hmac sha2_256 sha1

firewall detect ftp

nat server server6_all protocol tcp global 202.96.37.121 www inside 192.168.200.2 www

user-interface con 0

authentication-mode aaa

user-interface vty 0 4

authentication-mode aaa

protocol inbound ssh

user-interface vty 16 20

pki realm default

sa

location

nat address-group l 0

mode pat

nat address-group 1 1

mode pat

section 0 202.96.37.88 202.96.37.88

nat address-group 0 2

mode pat

section 0 202.96.37.20 202.96.37.30

multi-linkif

mode proportion-of-weight

right-manager server-group

device-classification

device-group pc

device-group mobile-terminal

device-group undefined-group

user-manage server-sync tsm

security-policy

rule name trust-dmz

source-zone trust

destination-zone dmz

source-address 10.10.40.0 mask 255.255.255.0

destination-address 192.168.200.1 mask 255.255.255.255

action permit

rule name trust-untrust

source-zone trust

destination-zone untrust

source-address-exclude 10.10.70.0 mask 255.255.255.0

action permit

rule name untrust-dmz

source-zone untrust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

rule name local2any

source-zone local

action permit

rule name any2local

destination-zone local

action permit

auth-policy

traffic-policy

policy-based-route

nat-policy

rule name nat

source-zone trust

destination-zone untrust

source-address 10.10.0.0 mask 255.255.192.0

source-address 192.168.110.0 mask 255.255.255.0

source-address 192.168.120.0 mask 255.255.255.0

destination-address-exclude 10.20.0.0 mask 255.255.0.0

destination-address-exclude 10.30.0.0 mask 255.255.0.0

action source-nat address-group 0

quota-policy

pcp-policy

dns-transparent-policy

rightm-policy

return

AR2

[V200R003C00]

sysname ISP

board add 0/2 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface Ethernet0/0/0

interface Ethernet0/0/1

interface Ethernet0/0/2

interface Ethernet0/0/3

interface Ethernet0/0/4

interface Ethernet0/0/5

interface Ethernet0/0/6

interface Ethernet0/0/7

interface GigabitEthernet0/0/0

ip address 100.1.1.6 255.255.255.248

interface GigabitEthernet0/0/1

ip address 116.116.116.1 255.255.255.0

interface GigabitEthernet2/0/0

ip address 114.114.114.254 255.255.255.0

interface GigabitEthernet2/0/1

ip address 117.117.117.1 255.255.255.0

interface GigabitEthernet2/0/2

ip address 12.1.1.1 255.255.255.0

interface GigabitEthernet2/0/3

interface NULL0

ospf 1

import-route static

area 0.0.0.0

network 0.0.0.0 255.255.255.255

ip route-static 192.168.200.0 255.255.255.0 202.96.37.10

ip route-static 202.96.37.0 255.255.255.0 100.1.1.1

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

LSW41

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

服务器LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

联通LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

interface Ethernet0/0/1

port link-type access

interface GigabitEthernet0/0/1

port link-type access

user-interface con 0

user-interface vty 0 4

return

移动LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

AR5

[V200R003C00]

sysname isp-CM

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load portalpage.zip

drop illegal-mac alarm

set cpu-usage threshold 80 restore 75

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 115.115.115.254 255.255.255.0

interface GigabitEthernet0/0/1

ip address 2.2.2.6 255.255.255.248

interface GigabitEthernet0/0/2

ip address 12.1.1.2 255.255.255.0

ospf 1

import-route static

area 0.0.0.0

network 0.0.0.0 255.255.255.255

ip route-static 202.96.37.0 255.255.255.0 2.2.2.1

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

AR3

[V200R003C00]

board add 0/4 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

undo info-center enable

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

dhcp enable

acl number 2000

rule 5 permit source 10.20.10.0 0.0.0.255

rule 10 permit source 10.20.20.0 0.0.0.255

rule 15 permit source 10.20.30.0 0.0.0.255

ip pool vlan210

gateway-list 10.20.10.254

network 10.20.10.0 mask 255.255.255.0

excluded-ip-address 10.20.10.252 10.20.10.253

dns-list 114.114.114.114

ip pool vlan220

gateway-list 10.20.20.254

network 10.20.20.0 mask 255.255.255.0

excluded-ip-address 10.20.20.252 10.20.20.253

dns-list 114.114.114.114

ip pool vlan230

gateway-list 10.20.30.254

network 10.20.30.0 mask 255.255.255.0

excluded-ip-address 10.20.30.252 10.20.30.253

dns-list 114.114.114.114

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 116.116.116.2 255.255.255.0

nat outbound 2000

interface GigabitEthernet0/0/1

ip address 10.20.10.254 255.255.255.0

dhcp select global

interface GigabitEthernet0/0/2

ip address 10.20.20.254 255.255.255.0

dhcp select global

interface GigabitEthernet4/0/0

ip address 10.20.30.254 255.255.255.0

dhcp select global

interface Tunnel0/0/0

ip address 192.168.1.18 255.255.255.252

tunnel-protocol gre

source 116.116.116.2

destination 100.1.1.2

interface Tunnel0/0/1

ip address 192.168.1.25 255.255.255.252

tunnel-protocol gre

source 116.116.116.2

destination 117.117.117.2

ospf 1

area 0.0.0.0

ip route-static 0.0.0.0 0.0.0.0 116.116.116.1

ip route-static 10.10.0.0 255.255.0.0 192.168.1.17

ip route-static 10.30.0.0 255.255.0.0 192.168.1.26

ip route-static 192.168.0.0 255.255.0.0 192.168.1.17

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

AR4

[V200R003C00]

board add 0/4 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

dhcp enable

acl number 2000

rule 5 permit source 10.30.10.0 0.0.0.255

rule 10 permit source 10.30.20.0 0.0.0.255

rule 15 permit source 10.30.30.0 0.0.0.255

ip pool vlan310

gateway-list 10.30.10.254

network 10.30.10.0 mask 255.255.255.0

excluded-ip-address 10.30.10.252 10.30.10.253

dns-list 114.114.114.114

ip pool vlan320

gateway-list 10.30.20.254

network 10.30.20.0 mask 255.255.255.0

excluded-ip-address 10.30.20.252 10.30.20.253

dns-list 114.114.114.114

ip pool vlan330

gateway-list 10.30.30.254

network 10.30.30.0 mask 255.255.255.0

excluded-ip-address 10.30.30.252 10.30.30.253

dns-list 114.114.114.114

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 117.117.117.2 255.255.255.252

nat outbound 2000

interface GigabitEthernet0/0/1

ip address 10.30.10.254 255.255.255.0

dhcp select global

interface GigabitEthernet0/0/2

ip address 10.30.20.254 255.255.255.0

dhcp select global

interface GigabitEthernet4/0/0

ip address 10.30.30.254 255.255.255.0

dhcp select global

interface Tunnel0/0/0

ip address 192.168.1.22 255.255.255.252

tunnel-protocol gre

source 117.117.117.2

destination 2.2.2.2

interface Tunnel0/0/1

ip address 192.168.1.26 255.255.255.252

tunnel-protocol gre

source 117.117.117.2

destination 116.116.116.2

ospf 1

area 0.0.0.0

network 192.168.0.0 0.0.255.255

ip route-static 0.0.0.0 0.0.0.0 117.117.117.1

ip route-static 10.10.0.0 255.255.0.0 192.168.1.21

ip route-static 10.20.0.0 255.255.0.0 192.168.1.25

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

北校区教学楼LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

北校区宿舍楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

北校区食堂

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区教学楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区宿舍楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区食堂

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return