课程实验报告

2025-2026 学年 第二学期

| 课程名称： | 计算系统综合课程设计 |
| --- | --- |
| 专 业： | 计算机科学与技术 （校企深度融合） |
| 班 级： | 4231091104 |
| 姓 名： | 孟子诚 |
| 学 号： | 423109110501 |

信息工程学院

一． 园区网安全实训目的

1.掌握园区网中常见安全威胁（ARP欺骗、广播风暴、非法接入、DDoS等）的防御方法。

2.学习 VLAN 隔离、端口安全、ACL、防火墙策略及 IDS 部署等核心技术。

3.能够独立完成中小型园区网的安全方案设计、设备配置与测试验证。

4.提升网络故障排查与安全事件应急响应能力。

二. 园区网需求分析

    - 2.1 业务需求

1. VLAN与Trunk建设

将内网划分为多个VLAN，减小广播域。

交换机端口配置Access，允许VLAN通过。

交换机端口配置Trunk，允许多VLAN通过。

2. 网关与VLAN间路由

核心交换机配置各VLAN的网关，实现VLAN间路由。

3. DHCP自动分配

核心层为用户终端自动分配IP地址、网关、DNS。

4. 动态路由（OSPF）

所有校区之间运行动态路由协议，实现路由自动学习与链路冗余。

5. 出口多链路选路

至少两条出口链路，配置主备或负载均衡，单链路故障时自动切换。

6. NAT地址转换

内网用户访问互联网时，源地址转换为公网IP。

7. Web服务器映射

将内网Web服务器的80端口映射到公网，允许外网访问。

8. 设备远程管理

所有网络设备支持远程登录管理，使用统一认证账号。

9. 部分VLAN禁止上网

企业财务服务器，只允许（行政楼）的职工访问。其他职工禁止访问。

禁止二号学院楼的职工访问外网且关键设备做好实时监控

10.设备监控

设置管理口令和协议版本，以便网管软件能监控或配置设备。

    - 2.2 安全需求

1.防止外部网络非法访问内部服务器。

2.防止内部用户进行扫描、攻击或越权访问。

3.实现端口安全，限制每个接入端口最大 MAC 地址数。

4.记录关键流量日志，支持攻击检测与溯源。

    - 2.3 性能与可靠性

1.核心链路采用链路聚合与冗余设计。

2.核心交换机双机热备（VRRP/HSRP）。

三. 园区网拓扑结构设计

本园区网采用三层架构 + 独立服务器区 + 边界防火墙，拓扑图如下：

    - 3.1 拓扑说明

| 层级 | 设备 | 功能 |
| --- | --- | --- |
| 核心层 | LSW1 / LSW2 | 高速转发、VLAN 间路由、冗余 |
| 汇聚层 | 教学楼LSW | 区域流量汇聚、执行 ACL 策略 |
| 接入层 | 科研楼LSW，1号学院楼LSW，行政楼LSW,图书馆LSW,实验楼LSW,2号学院楼LSW | 用户接入、端口安全、VLAN 划分 |
| 边界安全 | FW-1 / FW-2（主备） | NAT、访问控制、VPN、攻击防护 |
| 服务器集群 | 行政楼财政服务器，web服务器，联通服务器，移动服务器 | 业务服务与数据存储 |

    - 3.2 IP 地址规划

| 区域 | VLAN | 网段 | 网关 |
| --- | --- | --- | --- |
| 教学楼区 | 10 | 10.10.10.0/24 | 10.10.10.254 |
| 科研楼区 | 20 | 10.10.20.0/24 | 10.10.20.254 |
| 1号学院楼区 | 30 | 10.10.30.0/24 | 10.10.30.254 |
| 行政楼区 | 40 | 10.10.40.0/24 | 10.10.40.254 |
| 图书馆区 | 50 | 10.10.50.0/24 | 10.10.50.254 |
| 实验楼区 | 60 | 10.10.60.0/24 | 10.10.60.254 |
| 2号学院楼区 | 70 | 10.10.70.0/24 | 10.10.70.254 |
| 服务器区 | 200 | 192.168.200.0/24 | 192.168.200.254 |
| 信息中心1区 | 110 | 192.168.110.0/24 | 192.168.110.254 |
| 信息中心2区 | 120 | 192.168.120.0/24 | 192.168.120.254 |
| 北校区教学楼区 | 210 | 10.20.10.0/24 | 10.20.10.254 |
| 北校区宿舍楼区 | 220 | 10.20.20.0/24 | 10.20.20.254 |
| 北校区食堂区 | 230 | 10.20.30.0/24 | 10.20.30.254 |
| 南校区教学楼区 | 310 | 10.30.10.0/24 | 10.30.10.254 |
| 南校区宿舍楼区 | 320 | 10.30.20.0/24 | 10.30.20.254 |
| 南校区食堂区 | 330 | 10.30.30.0/24 | 10.30.30.254 |

四. 网络设备选型

| 设备类型 | 型号 | 关键特性 |
| --- | --- | --- |
| 核心交换机 | S5700 | 高吞吐、冗余电源、支持 VRRP/IRF2 |
| 汇聚交换机 | S3700 | 三层功能、ACL、QoS、VLAN 间路由 |
| 接入交换机 | H3C S5120V2-28P | 千兆接入、端口安全、802.1X 认证 |
| 防火墙 | H3C SecPath F1070 | 应用识别、入侵防御、VPN、双机热备 |
| IDS | Snort + 日志服务器 | 开源入侵检测，规则定期更新 |
| 服务器 | 华为 FusionServer | 部署 Web、DB、文件、备份服务 |

五、园网络拓扑结构设计

    - 5.1 组网说明

本园区网络系统采用业界通用的三层网络架构体系，整体分为接入层、汇聚层、核心层，同时单独规划服务器专属区域与外网出口安全区域，搭配多项冗余与互联技术，构建高可用、高安全、可拓展的园区网络体系。方案通过GRE隧道技术完成多校区之间的跨网络互联互通，依托防火墙双机热备机制筑牢外网出口的稳定性，全方位兼顾网络的业务承载能力、安全防护能力、故障冗余能力以及后续迭代拓展能力，详细组网规划内容如下：

一、网络层级架构与设备部署方案

全网整体架构分为接入、汇聚、核心三层数据转发体系，同时配套划分服务器业务区、出口安全防护区、外网运营商接入区，形成分层分区、各司其职的组网结构。

接入层依据园区楼宇分布部署多台接入交换机，覆盖教学楼、科研楼、一号学院楼、行政楼、图书馆、实验楼、二号学院楼等所有功能楼宇，主要承担各楼宇终端设备的入网接入工作，同时负责对应区域的VLAN划分、端口安全管控等基础接入策略配置，规范终端入网规范。

汇聚层设备承接各楼宇接入交换机的上行流量，完成片区流量的集中汇总，是接入层与核心层的中间枢纽。该层级主要部署ACL访问控制、流量限速、流量管控等安全与调度策略，实现区域流量的精细化管理。

核心层部署两台高性能核心交换机（LSW1、LSW2），为全网核心转发节点。设备采用链路聚合与VRRP虚拟路由冗余技术搭建双机备份架构，彻底规避单点故障风险。核心层主要负责全网高速数据转发、VLAN间三层路由互通、全网DHCP地址分发、OSPF动态路由计算等核心业务，是整个园区网络的数据传输与业务调度中枢。

二、VLAN规划与二层组网设计

本次组网按照园区不同功能楼宇、不同业务类型差异化划分独立VLAN，通过分割网络广播域，有效抑制广播风暴滋生，提升全网二层网络的稳定性与运行效率。

二层端口采用差异化配置模式：接入交换机连接终端设备的端口统一配置为Access模式，并根据终端所属业务区域划分至对应VLAN；接入层上联汇聚层、汇聚层上联核心层的互联端口全部配置为Trunk透传模式，放行所有业务VLAN流量，保障同一VLAN下的终端设备可跨设备、跨楼宇正常二层通信。

为解决二层网络环路隐患，同时实现链路冗余备份与流量负载均衡，全网统一部署MSTP多生成树协议，在保障网络无环路运行的基础上，最大化利用全网物理链路资源，提升网络容错能力。

三、三层路由配置与多校区互联

园区内网全程采用OSPF动态路由协议实现全网路由互通，设定骨干区域为Area 0，各楼宇、各业务片区对应划分独立的非骨干区域。核心交换机、汇聚交换机、校区互联路由器等所有核心网络设备均纳入OSPF路由进程，可实现内网路由自动发现、自动学习与快速收敛，大幅简化静态路由的配置工作量，提升网络适配性。

针对主校区、北校区、南校区的跨地域组网需求，通过在公网基础上搭建GRE加密隧道，实现各校区私有内网之间的安全互通，突破公网传输限制，完成多校区内网资源的跨网共享与业务联动。

四、出口安全防护与链路冗余设计

网络外网出口部署两台防火墙（FW1、FW2），搭建双机热备冗余架构，实现设备级故障自动切换。当单台防火墙出现故障、宕机或维护时，全网外网流量可无缝切换至另一台正常设备，确保园区外网业务不中断，保障出口链路高可用。

防火墙作为园区内网与外网的边界安全网关，集成区域安全划分、精细化安全访问策略、NAT地址转换、服务器端口映射等核心功能。内网所有私有IP终端通过源NAT转换为公网IP地址，实现互联网访问；内网Web业务服务器通过目的NAT端口映射技术，对外暴露公网访问接口，可为外网用户提供专属服务。

园区外网同时接入联通、移动两大运营商专线，配置链路主备调度策略，形成链路级冗余备份。当单条运营商线路出现中断、卡顿等故障时，系统可自动切换至备用链路，保障外网访问的连续性与稳定性。

五、业务管控与安全策略部署

为保障核心数据安全，将财务服务器部署在独立的服务器专属区域，通过防火墙安全策略搭配交换机ACL访问控制规则，设置精细化访问权限，仅开放行政楼网段的访问权限，严禁其他区域终端越权访问财务核心设备，筑牢数据安全防线。

针对业务管控需求，单独为二号学院楼网段配置外网访问限制策略，禁止该区域终端接入互联网，满足专属区域的网络管控要求。全网所有接入交换机均启用端口安全功能，限制单端口最大接入MAC地址数量，有效杜绝非法终端、陌生设备私自入网，防范网络入侵与蹭网风险。

全网所有网络设备统一配置AAA身份认证、远程登录管理服务及SNMP网管监控协议，支持平台化统一远程运维、设备状态监控与操作日志溯源，大幅提升网络运维效率与整体安全管控水平。

六、IP地址分配规范

园区内网终端IP地址以DHCP自动分配为核心方式，由核心交换机统一部署全局DHCP地址池，针对不同VLAN网段精准分配对应IP地址、网关地址及DNS解析地址，实现终端入网自动化配置。

为保障核心业务稳定，服务器设备、网络核心网关、设备互联接口、设备管理地址等关键网络节点全部采用静态IP配置，固定地址信息，避免地址变动引发的网络故障。同时，设备点对点互联链路统一采用/30子网掩码，精准划分互联网段，最大化节约内网IP地址资源，实现地址资源合理利用。

·  5.2 IP地址规划

全网采用私有IP地址与公网IP地址混合规划方案。校园网内部用户采用私有IP地址，通过防火墙NAT地址转换访问互联网。各功能区域按照VLAN进行地址划分，具体规划如下：

| VLAN ID | 网段 | 网关 | 用途 | 所属区域 |
| --- | --- | --- | --- | --- |
| VLAN 10 | 10.10.10.0/24 | 10.10.10.254 | 教学楼用户终端 | 主校区 |
| VLAN 210 | 10.20.10.0/24 | 10.20.10.254 | 北校区教学楼 | 北校区 |
| VLAN 220 | 10.20.20.0/24 | 10.20.20.254 | 北校区宿舍楼 | 北校区 |
| VLAN 230 | 10.20.30.0/24 | 10.20.30.254 | 北校区食堂 | 北校区 |
| VLAN 310 | 10.30.10.0/24 | 10.30.10.254 | 南校区教学楼 | 南校区 |
| VLAN 320 | 10.30.20.0/24 | 10.30.20.254 | 南校区宿舍楼 | 南校区 |
| VLAN 330 | 10.30.30.0/24 | 10.30.30.254 | 南校区食堂 | 南校区 |
| VLAN 110 | 192.168.110.0/24 | 192.168.110.254 | 信息中心管理网段 | 主校区 |
| VLAN 200 | 192.168.200.0/24 | 192.168.200.254 | 内部服务器网段 | 主校区 |

网络互联地址采用/30子网掩码，每条点对点链路分配一个独立网段。互联地址规划如下：

| 互联链路 | IP网段 | 互联设备 |
| --- | --- | --- |
| 防火墙 FW1至FW2 | 1.1.1.0/30 | FW1 ↔ FW2 |
| 核心 LSW1至防火墙FW1 | 192.168.1.0/30 | LSW1 ↔ FW1 |
| 核心 LSW1至FW2 | 192.168.1.4/30 | LSW1 ↔ FW2 |
| 核心LSW2至FW1 | 192.168.1.8/30 | LSW2 ↔ FW1 |
| 核心LSW2至FW1 | 192.168.1.12/30 | LSW2 ↔ FW2 |
| 信息中心至LSW1 | 192.168.1.16/30 | LSW32 ↔ FW1 |
| 信息中心至LSW1 | 192.168.1.20/30 | LSW32 ↔ FW2 |
| 信息中心至LSW2 | 192.168.1.24/30 | LSW34 ↔ FW1 |
| 信息中心至LSW2 | 192.168.1.28/30 | LSW34 ↔ FW2 |

公网出口地址规划：联通出口网段116.116.116.0/24，移动出口网段117.117.117.0/24。出口路由器与运营商之间运行OSPF实现路由可达。

DHCP地址分配策略：终端用户PC默认通过DHCP自动获取IP地址；服务器及关键管理设备（PC1等）采用静态IP地址配置。

DHCP服务器部署在核心层，为各VLAN提供地址分配服务，同时下发默认网关及DNS服务器地址。

- 关键设备配置

完整配置命令详见附录B。

    - 6.1 VLAN与Trunk配置

在核心交换机（LSW1/LSW2）及各接入交换机上创建业务VLAN，实现二层网络隔离。接入交换机与核心交换机之间配置Trunk链路，允许所有业务VLAN通过。以主校区教学楼区域为例：

【核心交换机LSW1关键配置】

vlan batch 10 20 21 22 30 31 32 110 120 200

interface GigabitEthernet0/0/1  # 上联FW1

port link-type access

port default vlan 110

interface GigabitEthernet0/0/2  # 上联FW2

port link-type access

port default vlan 110

interface GigabitEthernet0/0/3  # 互联LSW2

port link-type trunk

port trunk allow-pass vlan all

interface GigabitEthernet0/0/4  # 下联教学楼LSW

port link-type trunk

port trunk allow-pass vlan 120

# 配置VLANIF接口作为各VLAN网关

interface Vlanif10

ip address 10.10.10.254 255.255.255.0

interface Vlanif110

ip address 192.168.110.254 255.255.255.0

interface Vlanif120

ip address 192.168.120.254 255.255.255.0

    - 6.2 DHCP自动分配配置

在核心交换机上启用DHCP服务，为各VLAN终端自动分配IP地址。配置全局DHCP地址池及接口DHCP模式：

dhcp enable

ip pool vlan10

network 10.10.10.0 mask 255.255.255.0

gateway-list 10.10.10.254

dns-list 115.115.115.115

interface Vlanif10

dhcp select global

ip pool vlan120

network 192.168.120.0 mask 255.255.255.0

gateway-list 192.168.120.254

dns-list 115.115.115.115

interface Vlanif120

dhcp select global

    - 6.3 OSPF动态路由配置

启用OSPF动态路由协议，区域0为骨干区域，实现路由自动学习与快速收敛。核心交换机、汇聚交换机、路由器均参与OSPF路由域：

ospf 1 router-id 1.1.1.1

area 0.0.0.0

network 192.168.110.0 0.0.0.255

network 10.10.10.0 0.0.0.255

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

# 出口路由器引入默认路由

ospf 1

default-route-advertise always

    - 6.4 NAT地址转换配置

本次园区网采用源 NAT（PAT 多对多转换） 实现内网用户访问互联网，搭配目的 NAT（服务器端口映射） 实现内网 Web 服务器对外发布，同时结合多出口链路、安全策略完成 NAT 部署，以下基于 FW1 设备配置说明，FW2 配置逻辑一致。

1. 配置 NAT 地址组

定义公网地址池，工作模式为 PAT，支持多终端共用公网 IP 访问外网。

plaintext

nat address-group 0 2

mode pat

section 0 202.96.37.20 202.96.37.30

nat address-group 1 1

mode pat

section 0 202.96.37.88 202.96.37.88

2. 配置源 NAT 策略

匹配内网信任区域访问外网非信任区域的流量，对指定内网网段进行源地址转换，同时排除南北校区跨校区流量，避免误转换。

plaintext

nat-policy

rule name nat

source-zone trust

destination-zone untrust

source-address 10.10.0.0 mask 255.255.192.0

source-address 192.168.110.0 mask 255.255.255.0

source-address 192.168.120.0 mask 255.255.255.0

destination-address-exclude 10.20.0.0 mask 255.255.0.0

destination-address-exclude 10.30.0.0 mask 255.255.0.0

action source-nat address-group 0

3. 配置服务器端口映射（目的 NAT）

将 DMZ 区域内网 Web 服务器的 80 端口映射至公网地址，实现外网用户访问内网业务服务。

plaintext

nat server server6_all protocol tcp global 202.96.37.121 www inside 192.168.200.2 www

    - 6.5 防火墙双机热备配置

采用HRP+VRRP实现两台防火墙主备冗余，设备故障时流量自动切换。

1. 主防火墙 FW1 配置

plaintext

hrp enable

hrp interface GigabitEthernet1/0/0 remote 1.1.1.2

hrp mirror session enable

interface GigabitEthernet1/0/3

vrrp vrid 2 virtual-ip 192.168.200.254 active

interface GigabitEthernet1/0/4

vrrp vrid 1 virtual-ip 100.1.1.1 active

interface GigabitEthernet1/0/5

vrrp vrid 2 virtual-ip 2.2.2.1 active

2. 简要说明

全局启用 HRP 建立双机备份、同步会话表；各接口配置 VRRP 虚拟网关，FW1 作为主设备。FW2 对应接口 VRRP 状态为standby，为主设备故障时的备用网关，实现设备与链路双重热备。

七、网络连通性测试

    - 7.1 教一楼ping教二楼

测试目的：验证同一VLAN内终端之间的二层通信正常，在VLAN10上ping同VLAN10内其他终端，验证二层可达性。预期结果：同VLAN内PC之间可以相互ping通，且ARP表项正常学习。

    - 7.2 行政楼ping信息中心PC1

测试目的：验证不同VLAN之间通过核心交换机VLANIF三层接口实现互通。从VLAN40（行政楼）的PC ping VLAN110（信息中心）的终端，验证跨VLAN三层通信。预期结果：核心交换机上VLANIF接口状态UP，跨VLAN Ping成功。

    - 7.3 行政楼DHCP获取ip地址

测试目的：验证终端PC能够通过DHCP自动获取IP地址、子网掩码、默认网关及DNS服务器信息。将PC设置为DHCP模式，执行ipconfig 命令。预期结果：终端成功获取对应VLAN网段的IP地址及网络参数。

    - 7.4 行政楼ping联通服务器

测试目的：验证内网能否ping通外网

    - 7.5 行政楼ping北校区

测试目的：验证主校区与北校区之间通过GRE隧道实现跨Internet互通。

    - 7.6 行政楼ping北校区教学楼

测试目的：验证FW1与FW2之间的双机热备切换功能。手动断开FW1的上行链路，观察FW2是否自动接管流量，业务中断时间是否在秒级范围内。预期结果：FW2自动切换为主设备，业务中断时间<3秒，切换后内网用户上网不受影响。

    - 7.7 行政楼财政服务器

行政楼ping财政服务器。

测试目的：验证行政楼能否访问行政楼服务器。

内网其余ping财政服务器。

测试目的：验证其余能否访问财政服务器。

外网ping财政服务器。

测试目的：验证外网能否访问财政服务器。

    - 7.8 服务器80端口访问

外网访问80端口。

测试目的：外网可以正常访问80端口。

内网访问80端口。

测试目的：内网可以正常访问80端口。

PC机ping服务器。

测试目的：只能访问80端口，其他端口应不能访问。

预测结果：ping超时。

    - 7.9 2号学院楼ping外网

测试目的：测试2号学院楼能否访问外网。

预测结果：ping超时。

    - 7.10 设备监控

- 网络安全分析

    - 8.1 网络整体现状概述

本次园区网采用三层架构搭建，划分多 VLAN 实现二层隔离，搭配防火墙、接入安全策略、动态路由、GRE 隧道等技术，完成主校区、北校区、南校区多区域互联。网络部署了 VLAN 隔离、端口安全、防火墙访问控制、NAT 转换、双机热备、远程管理认证等基础安全机制，同时通过 OSPF、VRRP、链路聚合实现网络冗余，保障业务连续运行。整体网络可满足日常办公、上网、内网服务发布、跨校区通信等基础业务需求，基础安全防护体系已初步建成，能够抵御常规的非法接入、跨网段越权访问等简单威胁。

    - 8.2 远程登录

本次网络设备开启远程登录功能，支持Telnet协议，结合设备配置与实际测试，对协议的安全性进行分析。

Telnet 属于明文传输协议，设备登录账号、密码、配置指令等所有数据均以明文形式在网络中转发。在ARP中间人欺骗测试环境下，攻击者可轻松截获Telnet登录凭证，直接获取设备管理员权限，篡改网络配置、关闭安全策略，对整网造成严重破坏。

    - 8.3 已发生安全事件专项分析

结合本次连通性与功能测试过程，模拟并观测到几类典型网络异常与安全事件：

1.DHCP 耗尽攻击测试

攻击操作：在 Kali 主机执行dhcpstarv -i eth5 -v 10.10.20.1-254，工具持续向网段 DHCP 服务器大批量发送伪造 DHCP 请求报文，疯狂抢占地址池内可用 IP。

攻击现象：日志持续输出分配记录，大量虚拟 MAC 地址从 DHCP 服务器 10.10.20.254 获取 10.10.20 网段 IP 地址；服务器仅下发 IP、缺失子网掩码配置项。短时间内地址池可用地址被虚假终端大量占用，合法后续接入设备无法获取 IP，出现内网断网、入网失败故障。

风险危害：属于拒绝服务攻击，直接瘫痪网段 DHCP 分配服务，阻止正常办公终端、业务设备接入网络，大面积影响内网基础连通。

2.ARP 双向欺骗中间人攻击测试

攻击操作：向终端 10.10.20.251 发送伪造 ARP 报文，谎称网关 10.10.20.254 的 MAC 为攻击机网卡 MAC；同步向网关发送伪造 ARP，谎称终端 IP 对应攻击机 MAC，实现流量劫持。

攻击现象：前期欺骗包持续发送，短暂无流量拦截；之后成功劫持双向流量，监控捕获终端与网关之间的传输数据包。攻击机作为中间人，可以窃听、篡改内网明文传输数据（账号、文件、网页内容等）。

被攻击后ping网关不通。

3.主机存活探测

攻击操作：扫描网段存活主机，通过 ICMP、ARP 扫描快速嗅探在线设备。

探测现象：扫描快速输出网段内所有开机终端、网关、服务器 IP 与对应 MAC 地址，攻击者可快速绘制内网资产拓扑，定位可攻击目标。

安全风险：为后续端口爆破、漏洞渗透、定向欺骗攻击提供精准目标情报，是内网渗透的前置侦察步骤。

- 端口探测

攻击操作：针对存活主机开展全端口扫描，识别开放服务端口。

探测现象：扫描结果列出目标开放端口、对应服务版本（如 80 Web、22 SSH、3389 远程桌面等）；攻击者依据开放端口匹配对应系统漏洞、弱口令爆破点。

安全风险：暴露设备对外业务端口与服务版本，极易引发漏洞利用、暴力破解、恶意入侵，威胁终端与服务器数据安全。

    - 8.4 网络现存主要安全风险

接入层安全风险：虽然配置端口安全限制接入终端数量，但未开启 802.1X 认证、MAC 地址绑定，存在私自更换终端、盗用合法端口接入非法设备的风险。

设备管理风险：全网设备统一使用固定管理账号与密码，口令复杂度虽达标，但未定期轮换，存在账号泄露、远程登录被暴力破解的隐患。

日志与审计缺失：设备仅开启基础 SNMP 监控，未部署独立日志服务器与行为审计系统，流量异常、违规操作无法完整记录，安全事件难以溯源。

边界防护短板：防火墙仅配置基础包过滤策略，未深度开启入侵防御、病毒查杀、URL 过滤功能，无法抵御木马、恶意脚本、网页攻击等应用层威胁。

GRE 隧道安全风险：校区间 GRE 隧道为明文传输，数据在公网转发过程中存在被窃听、篡改的风险。

广播域与环路隐患：虽部署 VLAN 与生成树协议，但未对广播风暴、异常组播流量做限流策略，大流量广播报文易造成网络卡顿。

    - 8.5 校园网面临的主要安全威胁类型

结合园区使用场景与网络架构，梳理出日常运行中易遭遇的安全威胁：

非法接入威胁：用户私自加装路由器、随身 WiFi、外来终端接入内网，挤占带宽同时引入外部病毒与恶意程序。

越权访问威胁：内网用户尝试访问财务服务器、管理网段等涉密区域，非法窃取内部数据。

网络攻击威胁：包括 ARP 欺骗、端口扫描、泛洪攻击，易造成网关欺骗、网络断网、带宽拥塞。

外网恶意威胁：外网木马、钓鱼网站、恶意攻击通过互联网渗透至内网，入侵终端与服务器。

数据传输威胁：跨校区 GRE 明文隧道传输业务数据，存在数据窃听、篡改风险。

人为操作威胁：员工误配置设备、违规使用下载软件、传播病毒文件，引发内网大面积故障。

    - 8.6 现有安全防护体系短板分析

防护层次单一：当前防护集中在二层隔离 + 三层包过滤，仅实现基础访问控制，缺乏应用层安全防护，对高级攻击防御能力不足。

安全设备功能未充分利用：防火墙仅启用 NAT、VRRP、基础安全策略，入侵防御、防病毒、内容过滤等安全模块处于闲置状态。

终端安全管控薄弱：安全策略仅部署在网络设备上，未对接终端安全软件，无法对终端行为、病毒、木马进行统一管控。

安全监控与预警能力不足：仅依靠 SNMP 查看设备运行状态，无实时流量监控、异常告警功能，安全问题出现后无法提前预警。

隧道与传输安全缺失：GRE 隧道未加密，公网传输数据无保护机制，跨校区数据安全性无法保障。

应急响应机制不完善：未制定安全事件应急处置流程，遭遇攻击、网络瘫痪时，故障排查与恢复效率较低。

    - 8.7 网络安全综合评估

结合本园区网络整体架构设计、安全配置部署、运行测试效果及现存风险隐患，对全网安全防护等级进行综合性研判，具体评估结果如下：

一、基础访问安全：合格

园区内网通过VLAN广播域分割、ACL访问控制策略、接入端口安全绑定、业务网段权限精细化划分等多重基础管控手段，实现了各功能区域、各业务网段的逻辑隔离与权限约束，可有效防范内网非法终端接入、跨区域越权访问等基础安全问题，能够满足内网基础访问安全规范要求。

二、设备与链路可靠性：良好

网络整体具备完善的冗余容错能力，外网出口防火墙采用双机热备架构，核心交换机部署VRRP虚拟路由冗余与链路聚合技术，同时配置双运营商出口主备链路。多重冗余机制彻底规避了单设备、单链路故障引发的网络中断问题，全网整体运行稳定性、业务持续性表现较好。

三、边界外网防护：基本合格

网络边界仅配置基础的包过滤访问策略完成外网防护，防护手段较为单一，缺乏完善的应用层安全防御能力，针对应用层渗透攻击、恶意流量入侵等高级威胁的拦截能力薄弱，外网边界存在一定的安全入侵风险，整体防护能力存在明显短板。

四、跨校区传输安全：不合格

现阶段多校区互联仅采用GRE隧道完成网络互通，隧道数据以明文形式传输，未配置加密防护机制，跨校区交互的业务数据、涉密数据无法保障保密性与完整性，存在数据窃听、信息泄露、报文篡改的安全隐患，跨网传输安全不达标。

五、安全审计与溯源能力：不合格

全网未部署独立的日志审计平台，缺乏统一的日志收集、存储与分析能力，网络设备登录操作、流量访问、异常攻击、策略触发等各类行为日志无法完整留存，安全事件发生后无法完成行为追溯、问题定位与事件复盘，不满足网络安全审计溯源的基本规范要求。

六、整体安全等级总结：基础可用，防护能力偏弱

本园区网络可保障日常基础业务正常运转，能够抵御常规、基础的网络安全威胁，满足基础网络安全运行标准。但整体安全体系防护维度不完善，针对复杂化、新型化的网络攻击及高级网络威胁抵御能力不足，距离标准化、规范化、立体化的智慧校园安全网络建设标准仍存在较大的优化与升级空间。

8.8 网络安全优化改进建议

为进一步补齐园区网络安全短板、夯实全网防护体系，针对终端接入、设备运维、边界防护、跨网传输、日志审计、终端安全及制度保障多个维度，制定全方位安全加固策略，具体优化内容如下：

一、强化接入层终端安全管控

针对网络接入层风险，对接入交换机实施端口与静态MAC地址绑定配置，同时联动802.1X端口身份认证机制，对所有入网终端进行身份核验，从源头拦截非法设备接入网络，杜绝私自入网、违规蹭网等安全隐患。与此同时，在全网接入设备中启用广播风暴抑制机制，对异常广播、组播流量进行阈值限制，有效规避端口环路、终端故障引发的全网广播风暴问题，保障二层网络稳定运行。

二、夯实网络设备管理安全基线

规范全网网络设备运维管理标准，建立定期的设备管理员密码更换机制，杜绝弱口令、长期不变口令带来的安全风险。针对运维人员权责，分级创建多权限管理账号，精细化划分设备配置、查询、运维等操作权限，实现权限分离、各司其职。同时收紧设备远程访问权限，关闭设备冗余、无用的远程登录服务，仅放行指定合规网段的设备进行远程运维，最大限度缩小设备暴露面，防止非法登录与越权操作。

三、升级边界防火墙综合防护能力

充分发挥出口防火墙的边界网关防护作用，全面开启入侵防御、病毒查杀、网页URL过滤、应用特征识别等高级安全防护功能，精准拦截网络木马程序、钓鱼恶意站点、漏洞攻击、异常渗透等各类网络威胁。优化边界安全策略规则，对内网访问外网、外网访问内网的双向流量进行精细化管控，按需放行合法业务流量，默认拦截异常、未知访问行为，筑牢网络边界安全屏障。

四、加固多校区跨网传输安全

在现有多校区GRE隧道跨网互联的基础上，叠加IPSec加密技术对隧道传输数据进行二次防护，对各校区之间交互的业务数据、传输报文进行加密封装。有效解决公网传输过程中的数据窃听、报文篡改、数据劫持等安全风险，实现跨校区数据安全、私密、可靠传输，保障多校区联动业务的数据完整性与保密性。

五、搭建全网日志审计与告警体系

部署专用独立日志审计服务器，统一收纳、存储、汇总全网各类网络设备的运行日志，涵盖流量转发记录、设备登录操作、安全策略命中、异常访问等全维度日志数据，实现全网安全事件可追溯、可查询、可复盘。同时开启全网设备异常状态告警机制，针对设备离线、流量异常、违规访问、策略异常等问题实时触发告警，便于运维人员及时发现、处置网络安全隐患。

六、完善全网多层安全防护架构

构建终端至全网边界的一体化防护体系，全网终端统一部署终端安全防护软件，实现终端病毒查杀、恶意程序拦截、终端上网行为管控、违规操作约束等全方位终端安全管理。结合网络原有层级分区架构，细化各层级安全防护策略，打造“终端接入-汇聚管控-核心调度-边界防护”的立体化多层安全防护体系，实现全网无死角安全防护。

七、健全安全管理制度与应急保障机制

配套完善规范化的网络安全运维管理制度，明确网络设备日常运维、账号口令管理、终端入网审批、安全策略变更等标准化流程，让日常网络安全管理有规可依。同时针对各类网络突发风险场景制定专项应急处置预案，涵盖网络攻击、全网断网、病毒大规模爆发、设备故障等突发事件，规范应急处置流程与操作步骤，提升运维团队的故障处置、风险应对能力，保障园区网络长期安全、稳定、可靠运行

参考文献

[1] 华为技术有限公司. Huawei S5700系列交换机产品文档[Z]. 2024.

[2] 华为技术有限公司. Huawei USG6000V系列防火墙管理员指南[Z]. 2023.

[3] 华为技术有限公司. eNSP网络仿真平台用户手册[Z]. 2024.

[4] 谢希仁. 计算机网络（第8版）[M]. 北京: 电子工业出版社, 2021.

[5] 石志国, 薛为民. 网络安全技术与应用实践[M]. 北京: 清华大学出版社, 2022.

[6] RFC 2328: OSPF Version 2[S]. IETF, 1998.

[7] RFC 2784: Generic Routing Encapsulation (GRE)[S]. IETF, 2000.

附录

    - 附录A 拓扑设备清单（部分）

| 序号 | 设备名称 | 设备型号 | 所属区域 |
| --- | --- | --- | --- |
| 1 | FW1 | USG6000V | 出口区域 |
| 2 | FW2 | USG6000V | 出口区域 |
| 3 | LSW1 | S5700 | 核心层 |
| 4 | LSW2 | S5700 | 核心层 |
| 5 | LSW32 | S5700 | 汇聚层 |
| 6 | LSW34 | S5700 | 汇聚层 |
| 7 | LSW41 | S5700 | 汇聚层 |
| 8 | LSW33 | S3700 | 汇聚层 |
| 9 | LSW15 | S3700 | 汇聚层 |
| 10 | LSW18 | S3700 | 汇聚层 |
| 11 | LSW19 | S3700 | 汇聚层 |
| 12 | 教学楼LSW | S3700 | 主校区汇聚层 |
| 13 | 办公楼LSW | S3700 | 主校区汇聚层 |
| 14 | 1号学院楼LSW | S3700 | 主校区汇聚层 |
| 15 | 2号学院楼LSW | S3700 | 主校区汇聚层 |
| 16 | 图书馆LSW | S3700 | 主校区汇聚层 |
| 17 | 实验楼LSW | S3700 | 主校区汇聚层 |
| 18 | 教一LSW | S3700 | 主校区接入层 |
| 19 | 教二LSW | S3700 | 主校区接入层 |
| 20 | 北校区教学楼LSW | S3700 | 北校区汇聚层 |
| 21 | 北校区食堂LSW | S3700 | 北校区汇聚层 |
| 22 | 北校区宿舍楼LSW | S3700 | 北校区汇聚层 |
| 23 | 南校区教学楼LSW | S3700 | 南校区汇聚层 |
| 24 | 南校区食堂LSW | S3700 | 南校区汇聚层 |
| 25 | 北校区宿舍楼LSW | S3700 | 南校区汇聚层 |
| 26 | AR2 | AR1220 | 出口区域 |
| 27 | AR3 | AR2240 | 北校区核心 |
| 28 | AR4 | AR2240 | 南校区核心 |
| 29 | AR5 | AR2220 | 出口区域 |
| 30 | AR7 | AR2220 | 出口区域 |
| 31 | Server1 | Server | 服务器区 |
| 32 | Server3 | Server | 服务器区 |
| 33 | Server5 | Server | 服务器区 |
| 34 | Server6 | Server | 服务器区 |
| 35 | Client2 | Client | 北校区 |
| 36 | PC1 | PC | 信息中心 |

    - 附录B 配置命令汇总

教一LSW

sysname J1

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 10

interface Ethernet0/0/2

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/3

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCEC3794

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

教二LSW

sysname J2

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 10

interface Ethernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/3

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCE30444

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

教学楼LSW

sysname JX

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/3

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface Ethernet0/0/4

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCAC4BE6

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

科研楼LSW

sysname KY

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 20

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCA12D91

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

1号学院楼

sysname X1

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 30

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC036E4A

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

行政楼

sysname XZ

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 40

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCC74F7A

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

图书馆

sysname TS

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 50

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCFD1786

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

实验楼

sysname SY

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 60

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCE30657

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

2号学院楼

sysname X2

vlan batch 10 20 30 40 50 60 70 100

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 70

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 100

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC35019D

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

AR7

[V200R003C00]

sysname XXZX-PC

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load portalpage.zip

drop illegal-mac alarm

set cpu-usage threshold 80 restore 75

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 192.168.120.1 255.255.255.0

ip route-static 0.0.0.0 0.0.0.0 192.168.120.254

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

LSW18

sysname LSW18

vlan batch 110 120

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 110

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC8879DE

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW19

sysname LSW19

vlan batch 110 120

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Ethernet0/0/1

port link-type access

port default vlan 120

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC183B8C

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW32

sysname LSW32

undo info-center enable

vlan batch 105 to 108 110 120

stp instance 3 root primary

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif105

ip address 192.168.1.18 255.255.255.252

interface Vlanif106

ip address 192.168.1.22 255.255.255.252

interface Vlanif107

ip address 192.168.1.26 255.255.255.252

interface Vlanif108

ip address 192.168.1.30 255.255.255.252

interface Vlanif110

ip address 192.168.110.252 255.255.255.0

vrrp vrid 110 virtual-ip 192.168.110.254

vrrp vrid 110 priority 120

interface Vlanif120

ip address 192.168.120.252 255.255.255.0

vrrp vrid 120 virtual-ip 192.168.120.254

vrrp vrid 120 priority 120

interface Eth-Trunk2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 106

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 105

stp disable

interface GigabitEthernet0/0/3

eth-trunk 2

interface GigabitEthernet0/0/4

eth-trunk 2

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

area 0.0.0.2

network 192.168.110.0 0.0.0.255

network 192.168.1.16 0.0.0.3

network 192.168.1.20 0.0.0.3

network 192.168.120.0 0.0.0.255

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC900A19

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW34

sysname LSW34

undo info-center enable

vlan batch 105 to 108 110 120

stp instance 3 root secondary

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif105

ip address 192.168.1.18 255.255.255.252

interface Vlanif106

ip address 192.168.1.22 255.255.255.252

interface Vlanif107

ip address 192.168.1.26 255.255.255.252

interface Vlanif108

ip address 192.168.1.30 255.255.255.252

interface Vlanif110

ip address 192.168.110.253 255.255.255.0

vrrp vrid 110 virtual-ip 192.168.110.254

vrrp vrid 110 priority 110

interface Vlanif120

ip address 192.168.120.253 255.255.255.0

vrrp vrid 120 virtual-ip 192.168.120.254

vrrp vrid 120 priority 110

interface Eth-Trunk2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 107

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 108

stp disable

interface GigabitEthernet0/0/3

eth-trunk 2

interface GigabitEthernet0/0/4

eth-trunk 2

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

default-route-advertise

area 0.0.0.2

network 192.168.120.0 0.0.0.255

network 192.168.1.24 0.0.0.3

network 192.168.1.28 0.0.0.3

network 192.168.110.0 0.0.0.255

snmp-agent

snmp-agent local-engineid 800007DB034C1FCCBC3689

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW1

sysname LSW1

undo info-center enable

vlan batch 10 20 30 40 50 60 70 100 to 108

stp instance 1 root primary

stp instance 2 root secondary

dhcp enable

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

ip pool vlan10

gateway-list 10.10.10.254

network 10.10.10.0 mask 255.255.255.0

excluded-ip-address 10.10.10.252 10.10.10.253

dns-list 114.114.114.114

ip pool vlan20

gateway-list 10.10.20.254

network 10.10.20.0 mask 255.255.255.0

excluded-ip-address 10.10.20.252 10.10.20.253

dns-list 114.114.114.114

ip pool vlan30

gateway-list 10.10.30.254

network 10.10.30.0 mask 255.255.255.0

excluded-ip-address 10.10.30.252 10.10.30.253

dns-list 114.114.114.114

ip pool vlan40

gateway-list 10.10.40.254

network 10.10.40.0 mask 255.255.255.0

excluded-ip-address 10.10.40.252 10.10.40.253

dns-list 114.114.114.114

ip pool vlan50

gateway-list 10.10.50.254

network 10.10.50.0 mask 255.255.255.0

excluded-ip-address 10.10.50.252 10.10.50.253

dns-list 114.114.114.114

ip pool vlan60

gateway-list 10.10.60.254

network 10.10.60.0 mask 255.255.255.0

excluded-ip-address 10.10.60.252 10.10.60.253

dns-list 114.114.114.114

ip pool vlan70

gateway-list 10.10.70.254

network 10.10.70.0 mask 255.255.255.0

excluded-ip-address 10.10.70.252 10.10.70.253

dns-list 114.114.114.114

ip pool vlan100

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

excluded-ip-address 192.168.100.252 192.168.100.253

dns-list 114.114.114.114

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif10

ip address 10.10.10.252 255.255.255.0

vrrp vrid 10 virtual-ip 10.10.10.254

vrrp vrid 10 priority 120

dhcp select global

interface Vlanif20

ip address 10.10.20.252 255.255.255.0

vrrp vrid 20 virtual-ip 10.10.20.254

vrrp vrid 20 priority 120

dhcp select global

interface Vlanif30

ip address 10.10.30.252 255.255.255.0

vrrp vrid 30 virtual-ip 10.10.30.254

vrrp vrid 30 priority 120

dhcp select global

interface Vlanif40

ip address 10.10.40.252 255.255.255.0

vrrp vrid 40 virtual-ip 10.10.40.254

vrrp vrid 40 priority 110

dhcp select global

interface Vlanif50

ip address 10.10.50.252 255.255.255.0

vrrp vrid 50 virtual-ip 10.10.50.254

vrrp vrid 50 priority 110

dhcp select global

interface Vlanif60

ip address 10.10.60.252 255.255.255.0

vrrp vrid 60 virtual-ip 10.10.60.254

vrrp vrid 60 priority 110

dhcp select global

interface Vlanif70

ip address 10.10.70.252 255.255.255.0

vrrp vrid 70 virtual-ip 10.10.70.254

vrrp vrid 70 priority 110

dhcp select global

interface Vlanif100

ip address 192.168.100.252 255.255.255.0

vrrp vrid 100 virtual-ip 192.168.100.254

vrrp vrid 100 priority 120

dhcp select global

interface Vlanif101

ip address 192.168.1.1 255.255.255.252

interface Vlanif102

ip address 192.168.1.5 255.255.255.252

interface Vlanif106

ip address 192.168.1.21 255.255.255.252

interface Vlanif107

ip address 192.168.1.25 255.255.255.252

interface Eth-Trunk1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 101

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 102

stp disable

interface GigabitEthernet0/0/3

eth-trunk 1

interface GigabitEthernet0/0/4

eth-trunk 1

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/7

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/8

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/9

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/12

port link-type access

port default vlan 106

stp disable

interface GigabitEthernet0/0/13

port link-type access

port default vlan 107

stp disable

ospf 1

area 0.0.0.0

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

network 0.0.0.0 255.255.255.255

area 0.0.0.1

network 10.10.10.0 0.0.0.255

network 10.10.20.0 0.0.0.255

network 10.10.30.0 0.0.0.255

network 10.10.40.0 0.0.0.255

network 10.10.50.0 0.0.0.255

network 10.10.60.0 0.0.0.255

network 10.10.70.0 0.0.0.255

network 192.168.100.0 0.0.0.255

area 0.0.0.2

network 192.168.1.20 0.0.0.3

network 192.168.1.24 0.0.0.3

ospf 2

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC82448F

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

LSW2

sysname LSW2

undo info-center enable

vlan batch 10 20 30 40 50 60 70 100 to 108

stp instance 1 root secondary

stp instance 2 root primary

dhcp enable

stp region-configuration

region-name huawei

revision-level 1

instance 1 vlan 10 20 30 100 to 102

instance 2 vlan 40 50 60 70 103 to 104

instance 3 vlan 105 to 108 110 120

active region-configuration

ip pool vlan10

gateway-list 10.10.10.254

network 10.10.10.0 mask 255.255.255.0

excluded-ip-address 10.10.10.252 10.10.10.253

dns-list 114.114.114.114

ip pool vlan20

gateway-list 10.10.20.254

network 10.10.20.0 mask 255.255.255.0

excluded-ip-address 10.10.20.252 10.10.20.253

dns-list 114.114.114.114

ip pool vlan30

gateway-list 10.10.30.254

network 10.10.30.0 mask 255.255.255.0

excluded-ip-address 10.10.30.252 10.10.30.253

dns-list 114.114.114.114

ip pool vlan40

gateway-list 10.10.40.254

network 10.10.40.0 mask 255.255.255.0

excluded-ip-address 10.10.40.252 10.10.40.253

dns-list 114.114.114.114

ip pool vlan50

gateway-list 10.10.50.254

network 10.10.50.0 mask 255.255.255.0

excluded-ip-address 10.10.50.252 10.10.50.253

dns-list 114.114.114.114

ip pool vlan60

gateway-list 10.10.60.254

network 10.10.60.0 mask 255.255.255.0

excluded-ip-address 10.10.60.252 10.10.60.253

dns-list 114.114.114.114

ip pool vlan70

gateway-list 10.10.70.254

network 10.10.70.0 mask 255.255.255.0

excluded-ip-address 10.10.70.252 10.10.70.253

dns-list 114.114.114.114

ip pool vlan100

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

excluded-ip-address 192.168.100.252 192.168.100.253

dns-list 114.114.114.114

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher <K.R)YFE!!(I\I9%HS7.!Q!!

local-user admin privilege level 15

local-user admin service-type telnet

interface Vlanif10

ip address 10.10.10.253 255.255.255.0

vrrp vrid 10 virtual-ip 10.10.10.254

vrrp vrid 10 priority 110

dhcp select global

interface Vlanif20

ip address 10.10.20.253 255.255.255.0

vrrp vrid 20 virtual-ip 10.10.20.254

vrrp vrid 20 priority 110

dhcp select global

interface Vlanif30

ip address 10.10.30.253 255.255.255.0

vrrp vrid 30 virtual-ip 10.10.30.254

vrrp vrid 30 priority 110

dhcp select global

interface Vlanif40

ip address 10.10.40.253 255.255.255.0

vrrp vrid 40 virtual-ip 10.10.40.254

vrrp vrid 40 priority 120

dhcp select global

interface Vlanif50

ip address 10.10.50.253 255.255.255.0

vrrp vrid 50 virtual-ip 10.10.50.254

vrrp vrid 50 priority 120

dhcp select global

interface Vlanif60

ip address 10.10.60.253 255.255.255.0

vrrp vrid 60 virtual-ip 10.10.60.254

vrrp vrid 60 priority 120

dhcp select global

interface Vlanif70

ip address 10.10.70.253 255.255.255.0

vrrp vrid 70 virtual-ip 10.10.70.254

vrrp vrid 70 priority 120

dhcp select global

interface Vlanif100

ip address 192.168.100.253 255.255.255.0

vrrp vrid 100 virtual-ip 192.168.100.254

vrrp vrid 100 priority 110

dhcp select global

interface Vlanif103

ip address 192.168.1.9 255.255.255.252

interface Vlanif104

ip address 192.168.1.13 255.255.255.252

interface Vlanif105

ip address 192.168.1.17 255.255.255.252

interface Vlanif108

ip address 192.168.1.29 255.255.255.252

interface Eth-Trunk1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/1

port link-type access

port default vlan 104

stp disable

interface GigabitEthernet0/0/2

port link-type access

port default vlan 103

stp disable

interface GigabitEthernet0/0/3

eth-trunk 1

interface GigabitEthernet0/0/4

eth-trunk 1

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/7

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/8

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/9

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/12

port link-type access

port default vlan 105

stp disable

interface GigabitEthernet0/0/13

port link-type access

port default vlan 108

stp disable

interface GigabitEthernet0/0/14

port link-type trunk

port trunk allow-pass vlan 2 to 4094

ospf 1

area 0.0.0.0

network 192.168.1.8 0.0.0.3

network 192.168.1.12 0.0.0.3

area 0.0.0.1

network 10.10.10.0 0.0.0.255

network 10.10.20.0 0.0.0.255

network 10.10.30.0 0.0.0.255

network 10.10.40.0 0.0.0.255

network 10.10.50.0 0.0.0.255

network 10.10.60.0 0.0.0.255

network 10.10.70.0 0.0.0.255

network 192.168.100.0 0.0.0.255

area 0.0.0.2

network 192.168.1.16 0.0.0.3

network 192.168.1.28 0.0.0.3

snmp-agent

snmp-agent local-engineid 800007DB034C1FCC416636

snmp-agent community read  one@123

snmp-agent sys-info version v2c v3

snmp-agent trap enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

protocol inbound all

return

FW1

sysname FW1

l2tp domain suffix-separator @

undo info-center enable

ipsec sha2 compatible enable

undo telnet ipv6 server enable

clock timezone UTC add 00:00:00

hrp enable

hrp interface GigabitEthernet1/0/0 remote 1.1.1.2

hrp mirror session enable

hrp load balance device

update schedule location-sdb weekly Sun 07:23

firewall defend action discard

banner enable

user-manage web-authentication security port 8887

undo privacy-statement english

undo privacy-statement chinese

page-setting

user-manage security version tlsv1.1 tlsv1.2

password-policy

level high

user-manage single-sign-on ad

user-manage single-sign-on tsm

user-manage single-sign-on radius

user-manage auto-sync online-user

web-manager security version tlsv1.1 tlsv1.2

web-manager enable

web-manager security enable

firewall dataplane to manageplane application-apperceive default-action drop

undo ips log merge enable

decoding uri-cache disable

update schedule ips-sdb daily 02:04

update schedule av-sdb daily 02:04

update schedule sa-sdb daily 02:04

update schedule cnc daily 02:04

update schedule file-reputation daily 02:04

ip vpn-instance default

ipv4-family

time-range worktime

period-range 08:00:00 to 18:00:00 working-day

ike proposal default

encryption-algorithm aes-256 aes-192 aes-128

dh group14

authentication-algorithm sha2-512 sha2-384 sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

aaa

authentication-scheme default

authentication-scheme admin_local

authentication-scheme admin_radius_local

authentication-scheme admin_hwtacacs_local

authentication-scheme admin_ad_local

authentication-scheme admin_ldap_local

authentication-scheme admin_radius

authentication-scheme admin_hwtacacs

authentication-scheme admin_ad

authorization-scheme default

accounting-scheme default

domain default

service-type internetaccess ssl-vpn l2tp ike

internet-access mode password

reference user current-domain

manager-user audit-admin

password cipher @%@%|6S9U>C3D%*9!eN,94t/2D-XuqMS8igj9Xu@^s0fq#&LD-[2@%@%

service-type web terminal

level 15

manager-user api-admin

password cipher @%@%Id]R<~fBU=6(&s$Ccb#NTgM5[qv+&%>W<M+TQ+7/r|9XgM8T@%@%

level 15

manager-user admin

password cipher @%@%KC"',0gD~56ecF@@L'!ItnY{kFUJU6Hm`~(x$[J\TB!1nY!t@%@%

service-type web terminal telnet

level 15

role system-admin

role device-admin

role device-admin(monitor)

role audit-admin

bind manager-user audit-admin role audit-admin

bind manager-user admin role system-admin

l2tp-group default-lns

interface GigabitEthernet0/0/0

undo shutdown

ip binding vpn-instance default

ip address 192.168.0.1 255.255.255.0

alias GE0/METH

interface GigabitEthernet1/0/0

undo shutdown

ip address 1.1.1.1 255.255.255.252

interface GigabitEthernet1/0/1

undo shutdown

ip address 192.168.1.2 255.255.255.252

interface GigabitEthernet1/0/2

undo shutdown

ip address 192.168.1.10 255.255.255.252

interface GigabitEthernet1/0/3

undo shutdown

ip address 192.168.200.252 255.255.255.0

vrrp vrid 2 virtual-ip 192.168.200.254 active

interface GigabitEthernet1/0/4

undo shutdown

ip address 100.1.1.2 255.255.255.248

vrrp vrid 1 virtual-ip 100.1.1.1 active

service-manage ping permit

interface GigabitEthernet1/0/5

undo shutdown

ip address 2.2.2.2 255.255.255.248

vrrp vrid 2 virtual-ip 2.2.2.1 active

interface GigabitEthernet1/0/6

undo shutdown

interface Virtual-if0

interface NULL0

interface Tunnel0

interface Tunnel1

ip address 192.168.1.17 255.255.255.252

tunnel-protocol gre

source 100.1.1.2

destination 116.116.116.2

service-manage ping permit

interface Tunnel2

ip address 192.168.1.21 255.255.255.252

tunnel-protocol gre

source 2.2.2.2

destination 117.117.117.2

service-manage ping permit

firewall zone local

set priority 100

firewall zone trust

set priority 85

add interface GigabitEthernet0/0/0

add interface GigabitEthernet1/0/1

add interface GigabitEthernet1/0/2

firewall zone untrust

set priority 5

add interface GigabitEthernet1/0/4

add interface GigabitEthernet1/0/5

add interface Tunnel1

add interface Tunnel2

firewall zone dmz

set priority 50

add interface GigabitEthernet1/0/0

add interface GigabitEthernet1/0/3

ospf 1

default-route-advertise always

silent-interface GigabitEthernet1/0/3

area 0.0.0.0

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

network 192.168.1.8 0.0.0.3

network 192.168.1.12 0.0.0.3

network 192.168.1.20 0.0.0.3

network 192.168.200.0 0.0.0.255

ip route-static 0.0.0.0 0.0.0.0 100.1.1.6

ip route-static 0.0.0.0 0.0.0.0 2.2.2.6 preference 80

ip route-static 10.20.0.0 255.255.0.0 192.168.1.18

ip route-static 10.30.0.0 255.255.0.0 192.168.1.18

snmp-agent

snmp-agent local-engineid 800007DB0300E0FC465C76

snmp-agent community read cipher %^%#J~o#R2]=m::30[Kf-re3waFY>3,#o=DGv}TnErkRZ=a}-kTZjV5(gG,077w4,9rPU_OTTR$*xoR%nQ,S%^%#

snmp-agent sys-info version v2c v3

snmp-agent trap enable

undo ssh server compatible-ssh1x enable

ssh authentication-type default password

ssh server cipher aes256_ctr aes128_ctr

ssh server hmac sha2_256 sha1

ssh client cipher aes256_ctr aes128_ctr

ssh client hmac sha2_256 sha1

firewall detect ftp

nat server server6_all protocol tcp global 202.96.37.121 www inside 192.168.200.2 www

user-interface con 0

authentication-mode aaa

user-interface vty 0 4

authentication-mode aaa

user-interface vty 16 20

pki realm default

sa

location

nat address-group l 0

mode pat

nat address-group 1 1

mode pat

section 0 202.96.37.88 202.96.37.88

nat address-group 0 2

mode pat

section 0 202.96.37.20 202.96.37.30

multi-linkif

mode proportion-of-weight

right-manager server-group

device-classification

device-group pc

device-group mobile-terminal

device-group undefined-group

user-manage server-sync tsm

security-policy

rule name trust-dmz

source-zone trust

destination-zone dmz

source-address 10.10.40.0 mask 255.255.255.0

destination-address 192.168.200.1 mask 255.255.255.255

action permit

rule name trust-untrust

source-zone trust

destination-zone untrust

source-address-exclude 10.10.70.0 mask 255.255.255.0

action permit

rule name untrust-dmz

source-zone untrust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

rule name local2any

source-zone local

action permit

rule name any2local

destination-zone local

action permit

rule name trust-dmz-server

source-zone trust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

auth-policy

traffic-policy

policy-based-route

nat-policy

rule name nat

source-zone trust

destination-zone untrust

source-address 10.10.0.0 mask 255.255.192.0

source-address 192.168.110.0 mask 255.255.255.0

source-address 192.168.120.0 mask 255.255.255.0

destination-address-exclude 10.20.0.0 mask 255.255.0.0

destination-address-exclude 10.30.0.0 mask 255.255.0.0

action source-nat address-group 0

rule name hairpin-snat

source-zone trust

destination-zone untrust

destination-address 202.96.37.121 mask 255.255.255.255

service http

action source-nat easy-ip

quota-policy

pcp-policy

dns-transparent-policy

rightm-policy

return

FW2

sysname FW2

l2tp domain suffix-separator @

undo info-center enable

ipsec sha2 compatible enable

undo telnet server enable

undo telnet ipv6 server enable

clock timezone UTC add 00:00:00

hrp enable

hrp interface GigabitEthernet1/0/0 remote 1.1.1.1

hrp mirror session enable

hrp load balance device

update schedule location-sdb weekly Sun 01:54

firewall defend action discard

banner enable

user-manage web-authentication security port 8887

undo privacy-statement english

undo privacy-statement chinese

page-setting

user-manage security version tlsv1.1 tlsv1.2

password-policy

level high

user-manage single-sign-on ad

user-manage single-sign-on tsm

user-manage single-sign-on radius

user-manage auto-sync online-user

web-manager security version tlsv1.1 tlsv1.2

web-manager enable

web-manager security enable

firewall dataplane to manageplane application-apperceive default-action drop

undo ips log merge enable

decoding uri-cache disable

update schedule ips-sdb daily 04:46

update schedule av-sdb daily 04:46

update schedule sa-sdb daily 04:46

update schedule cnc daily 04:46

update schedule file-reputation daily 04:46

ip vpn-instance default

ipv4-family

time-range worktime

period-range 08:00:00 to 18:00:00 working-day

ike proposal default

encryption-algorithm aes-256 aes-192 aes-128

dh group14

authentication-algorithm sha2-512 sha2-384 sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

aaa

authentication-scheme default

authentication-scheme admin_local

authentication-scheme admin_radius_local

authentication-scheme admin_hwtacacs_local

authentication-scheme admin_ad_local

authentication-scheme admin_ldap_local

authentication-scheme admin_radius

authentication-scheme admin_hwtacacs

authentication-scheme admin_ad

authorization-scheme default

accounting-scheme default

domain default

service-type internetaccess ssl-vpn l2tp ike

internet-access mode password

reference user current-domain

manager-user audit-admin

password cipher @%@%CrgBRB69+>ELs0RulZ[V.Q}Ljvoz14tFP1^}pQ%u}``UQ}O.@%@%

service-type web terminal

level 15

manager-user api-admin

password cipher @%@%bna.T5B!sIxA4t,)D^yQ<:e;6L'~Ef:=1HXOW\<>EUO,:e><@%@%

level 15

manager-user admin

password cipher @%@%euP1JP:N;;.:vh82al:$x~X`4R[yFeJP_YzVMU<91lk#~Xcx@%@%

service-type web terminal telnet

level 15

role system-admin

role device-admin

role device-admin(monitor)

role audit-admin

bind manager-user audit-admin role audit-admin

bind manager-user admin role system-admin

l2tp-group default-lns

interface GigabitEthernet0/0/0

undo shutdown

ip binding vpn-instance default

ip address 192.168.0.1 255.255.255.0

alias GE0/METH

interface GigabitEthernet1/0/0

undo shutdown

ip address 1.1.1.2 255.255.255.252

interface GigabitEthernet1/0/1

undo shutdown

ip address 192.168.1.6 255.255.255.252

interface GigabitEthernet1/0/2

undo shutdown

ip address 192.168.1.14 255.255.255.252

interface GigabitEthernet1/0/3

undo shutdown

ip address 192.168.200.253 255.255.255.0

vrrp vrid 2 virtual-ip 192.168.200.254 standby

service-manage ping permit

interface GigabitEthernet1/0/4

undo shutdown

ip address 100.1.1.3 255.255.255.248

vrrp vrid 1 virtual-ip 100.1.1.1 standby

service-manage ping permit

interface GigabitEthernet1/0/5

undo shutdown

ip address 2.2.2.3 255.255.255.248

vrrp vrid 1 virtual-ip 2.2.2.1 standby

interface GigabitEthernet1/0/6

undo shutdown

interface Virtual-if0

interface NULL0

interface Tunnel0

interface Tunnel1

ip address 192.168.1.17 255.255.255.252

tunnel-protocol gre

source 202.96.37.10

destination 116.116.116.2

service-manage ping permit

interface Tunnel2

ip address 192.168.1.21 255.255.255.252

tunnel-protocol gre

source 202.96.37.10

destination 117.117.117.2

service-manage ping permit

firewall zone local

set priority 100

firewall zone trust

set priority 85

add interface GigabitEthernet0/0/0

add interface GigabitEthernet1/0/1

add interface GigabitEthernet1/0/2

firewall zone untrust

set priority 5

add interface GigabitEthernet1/0/4

add interface GigabitEthernet1/0/5

add interface Tunnel1

add interface Tunnel2

firewall zone dmz

set priority 50

add interface GigabitEthernet1/0/0

add interface GigabitEthernet1/0/3

ospf 1

default-route-advertise always

import-route static

silent-interface GigabitEthernet1/0/3

area 0.0.0.0

network 192.168.1.4 0.0.0.3

network 192.168.1.12 0.0.0.3

network 192.168.200.0 0.0.0.255

ip route-static 0.0.0.0 0.0.0.0 202.96.37.3

ip route-static 0.0.0.0 0.0.0.0 100.1.1.6

ip route-static 0.0.0.0 0.0.0.0 2.2.2.6 preference 80

ip route-static 10.20.0.0 255.255.0.0 192.168.1.18

ip route-static 10.30.0.0 255.255.0.0 192.168.1.22

snmp-agent

snmp-agent local-engineid 800007DB0300E0FC657F13

snmp-agent community read cipher %^%#r*dK@5*1AG%nnV9AmI':S$p(3X^I#Q,+"v'XbuiU2C~5V/3xL)l+';+N{u6V+1a\VJ'-W&(KR6<.0y_M%^%#

snmp-agent sys-info version v2c v3

snmp-agent trap enable

undo ssh server compatible-ssh1x enable

ssh authentication-type default password

ssh server cipher aes256_ctr aes128_ctr

ssh server hmac sha2_256 sha1

ssh client cipher aes256_ctr aes128_ctr

ssh client hmac sha2_256 sha1

firewall detect ftp

nat server server6_all protocol tcp global 202.96.37.121 www inside 192.168.200.2 www

user-interface con 0

authentication-mode aaa

user-interface vty 0 4

authentication-mode aaa

protocol inbound ssh

user-interface vty 16 20

pki realm default

sa

location

nat address-group l 0

mode pat

nat address-group 1 1

mode pat

section 0 202.96.37.88 202.96.37.88

nat address-group 0 2

mode pat

section 0 202.96.37.20 202.96.37.30

multi-linkif

mode proportion-of-weight

right-manager server-group

device-classification

device-group pc

device-group mobile-terminal

device-group undefined-group

user-manage server-sync tsm

security-policy

rule name trust-dmz

source-zone trust

destination-zone dmz

source-address 10.10.40.0 mask 255.255.255.0

destination-address 192.168.200.1 mask 255.255.255.255

action permit

rule name trust-untrust

source-zone trust

destination-zone untrust

source-address-exclude 10.10.70.0 mask 255.255.255.0

action permit

rule name untrust-dmz

source-zone untrust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

rule name local2any

source-zone local

action permit

rule name any2local

destination-zone local

action permit

rule name trust-dmz-server

source-zone trust

destination-zone dmz

destination-address 192.168.200.2 mask 255.255.255.255

service http

action permit

auth-policy

traffic-policy

policy-based-route

nat-policy

rule name nat

source-zone trust

destination-zone untrust

source-address 10.10.0.0 mask 255.255.192.0

source-address 192.168.110.0 mask 255.255.255.0

source-address 192.168.120.0 mask 255.255.255.0

destination-address-exclude 10.20.0.0 mask 255.255.0.0

destination-address-exclude 10.30.0.0 mask 255.255.0.0

action source-nat address-group 0

rule name hairpin-snat

source-zone trust

destination-zone untrust

destination-address 202.96.37.121 mask 255.255.255.255

service http

action source-nat easy-ip

quota-policy

pcp-policy

dns-transparent-policy

rightm-policy

return

AR2

sysname ISP

board add 0/2 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface Ethernet0/0/0

interface Ethernet0/0/1

interface Ethernet0/0/2

interface Ethernet0/0/3

interface Ethernet0/0/4

interface Ethernet0/0/5

interface Ethernet0/0/6

interface Ethernet0/0/7

interface GigabitEthernet0/0/0

ip address 100.1.1.6 255.255.255.248

interface GigabitEthernet0/0/1

ip address 116.116.116.1 255.255.255.0

interface GigabitEthernet2/0/0

ip address 114.114.114.254 255.255.255.0

interface GigabitEthernet2/0/1

ip address 117.117.117.1 255.255.255.0

interface GigabitEthernet2/0/2

ip address 12.1.1.1 255.255.255.0

interface GigabitEthernet2/0/3

interface NULL0

ospf 1

import-route static

area 0.0.0.0

network 0.0.0.0 255.255.255.255

ip route-static 192.168.200.0 255.255.255.0 202.96.37.10

ip route-static 202.96.37.0 255.255.255.0 100.1.1.1

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

LSW41

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

服务器LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

联通LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

interface Ethernet0/0/1

port link-type access

interface GigabitEthernet0/0/1

port link-type access

user-interface con 0

user-interface vty 0 4

return

移动LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

AR5

sysname isp-CM

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load portalpage.zip

drop illegal-mac alarm

set cpu-usage threshold 80 restore 75

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 115.115.115.254 255.255.255.0

interface GigabitEthernet0/0/1

ip address 2.2.2.6 255.255.255.248

interface GigabitEthernet0/0/2

ip address 12.1.1.2 255.255.255.0

ospf 1

import-route static

area 0.0.0.0

network 0.0.0.0 255.255.255.255

ip route-static 202.96.37.0 255.255.255.0 2.2.2.1

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

AR3

board add 0/4 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

undo info-center enable

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

dhcp enable

acl number 2000

rule 5 permit source 10.20.10.0 0.0.0.255

rule 10 permit source 10.20.20.0 0.0.0.255

rule 15 permit source 10.20.30.0 0.0.0.255

ip pool vlan210

gateway-list 10.20.10.254

network 10.20.10.0 mask 255.255.255.0

excluded-ip-address 10.20.10.252 10.20.10.253

dns-list 114.114.114.114

ip pool vlan220

gateway-list 10.20.20.254

network 10.20.20.0 mask 255.255.255.0

excluded-ip-address 10.20.20.252 10.20.20.253

dns-list 114.114.114.114

ip pool vlan230

gateway-list 10.20.30.254

network 10.20.30.0 mask 255.255.255.0

excluded-ip-address 10.20.30.252 10.20.30.253

dns-list 114.114.114.114

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 116.116.116.2 255.255.255.0

nat outbound 2000

interface GigabitEthernet0/0/1

ip address 10.20.10.254 255.255.255.0

dhcp select global

interface GigabitEthernet0/0/2

ip address 10.20.20.254 255.255.255.0

dhcp select global

interface GigabitEthernet4/0/0

ip address 10.20.30.254 255.255.255.0

dhcp select global

interface Tunnel0/0/0

ip address 192.168.1.18 255.255.255.252

tunnel-protocol gre

source 116.116.116.2

destination 100.1.1.2

interface Tunnel0/0/1

ip address 192.168.1.25 255.255.255.252

tunnel-protocol gre

source 116.116.116.2

destination 117.117.117.2

ospf 1

area 0.0.0.0

ip route-static 0.0.0.0 0.0.0.0 116.116.116.1

ip route-static 10.10.0.0 255.255.0.0 192.168.1.17

ip route-static 10.30.0.0 255.255.0.0 192.168.1.26

ip route-static 192.168.0.0 255.255.0.0 192.168.1.17

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

AR4

board add 0/4 4GET

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarm

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

dhcp enable

acl number 2000

rule 5 permit source 10.30.10.0 0.0.0.255

rule 10 permit source 10.30.20.0 0.0.0.255

rule 15 permit source 10.30.30.0 0.0.0.255

ip pool vlan310

gateway-list 10.30.10.254

network 10.30.10.0 mask 255.255.255.0

excluded-ip-address 10.30.10.252 10.30.10.253

dns-list 114.114.114.114

ip pool vlan320

gateway-list 10.30.20.254

network 10.30.20.0 mask 255.255.255.0

excluded-ip-address 10.30.20.252 10.30.20.253

dns-list 114.114.114.114

ip pool vlan330

gateway-list 10.30.30.254

network 10.30.30.0 mask 255.255.255.0

excluded-ip-address 10.30.30.252 10.30.30.253

dns-list 114.114.114.114

aaa

local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$

local-user admin service-type http

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 117.117.117.2 255.255.255.252

nat outbound 2000

interface GigabitEthernet0/0/1

ip address 10.30.10.254 255.255.255.0

dhcp select global

interface GigabitEthernet0/0/2

ip address 10.30.20.254 255.255.255.0

dhcp select global

interface GigabitEthernet4/0/0

ip address 10.30.30.254 255.255.255.0

dhcp select global

interface Tunnel0/0/0

ip address 192.168.1.22 255.255.255.252

tunnel-protocol gre

source 117.117.117.2

destination 2.2.2.2

interface Tunnel0/0/1

ip address 192.168.1.26 255.255.255.252

tunnel-protocol gre

source 117.117.117.2

destination 116.116.116.2

ospf 1

area 0.0.0.0

network 192.168.0.0 0.0.255.255

ip route-static 0.0.0.0 0.0.0.0 117.117.117.1

ip route-static 10.10.0.0 255.255.0.0 192.168.1.21

ip route-static 10.20.0.0 255.255.0.0 192.168.1.25

user-interface con 0

authentication-mode password

user-interface vty 0 4

user-interface vty 16 20

wlan ac

return

北校区教学楼LSW

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

北校区宿舍楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

北校区食堂

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区教学楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区宿舍楼

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return

南校区食堂

sysname Huawei

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

aaa

local-user admin password simple admin

local-user admin service-type http

user-interface con 0

user-interface vty 0 4

return